# Ignition! Notes

An Informal History of Liquid Rocket Propellants

Christopher Kalitin

For image references, Archive: https://archive.org/details/ignition_201612

| IX / X XI XII XIII XIV 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 66 67 68 69 70 71 72 73 74 75 76 77 78 79 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 100 101 102 103 104 105 106 107 108 109 110 111 112 113 114 115 116 117 118 119 120 121 122 123 124 125 126 127 128 129 130 131 132 133 134 135 136 137 138 139 140 141 142 143 144 145 146 147 148 149 150 151 152 153 154 155 156 157 158 159 160 161 162 163 164 165 166 167 168 169 170 171 172 173 174 175 176 177 178 179 180 181 182 183 184 185 186 187 188 189 190 191 192 | In Re John D. Clark: Asimov was a Chemist, Clark’s lab never had an accident. # Preface Liquid propellant work mostly active during late 40s, 50s, and first half of 60s. Trickle off after. Only in late 50s information on Soviet propellant work became available. Propellant community was at most ~200 people. Three quarters of which did what the other quarter told them to do. Many individuals. And informally spreading information in bars at big propellant meetings. Amazing group, I’m almost at the verge of tears. # How It Started Exploration of Space with Reactive Devices, Konstantin Eduardovitch Tsiolkovsky, 1903 5 primary points. Space travel is possible, only by means of rocket propulsion. Gunpowder rockets don’t have enough energy, certain liquids do, liquid hydrogen and liquid oxygen is a nearly ideal propellant combination. Wow holy shit. Starting with Micheal Faraday in 1823, scientists across Europe tried to convert common gasses into liquids. Cooling and compressing and both. Chlorine was first, then ammonia, CO2. By the 70s only oxygen, hydrogen, and nitrogen (fluorine still wasn’t concentrated enough in gaseous form). Wroblewski and Krakow in Austrian Poland made liquids of the remaining atoms. Liquid Air came in 1885 by them, 2 years after. By 1891, LOX in experimental quantities, by 1895, Linda developed a practical, large-scale process for making liquid air from LOX and LON are obtained simply by fractional distillation. My god Liquid Hydrogen in May 1898. Only then could Tsiolkosvky write of liquid LH2 & LOX propulsion. He later looked into hydrocarbons and alcohols. But always LOX as the oxidizer. 1909, Robert H. Goddard (Physics professor at Clark University) realized that LOX and LH2 were ideal propellants as well. In 1922, he began testing with gasoline and LOX. Nov `23, first firing on test stand, Mar 16 1926 first flight of a liquid rocket of 184 feet and 2.5 seconds. He had a 1.3-1.4 mixture ratio (3 is ideal), maybe for low combustion temp. 170 seconds ISP (Mass Thrust / Mass flow per second). 1923, Transylvanian German Herman Obert wrote The Rocket into Planetary Space and surprisingly became a minor best seller. Some read Oberth’s work and founded Verein fur Raumschifart, Society for Space Travel, VfR. June 1927. British Interplanetary Society and American Interplanetary Society in 1930 and 1933. Robert Esnault-Pelteris immensely detailed Astronautique in 1930. Most important work. Fritz Lang made movie Frau in Mond, hired Oberth as technical adviser and he would be given funding for a rocket engine at the premiere. He didn’t have time for gasoline LOX, instead Carbon rods in aluminum tube with LOX. It was vertical with the output at the top. LOX was stored inside always, assumed constant rate of LOX consumption and Carbon burning. Would’ve definitely exploded if he fired it. First Hybrid. Clark says there are many other hilarious stories on the movie industry, Oberth, and rocketry. VfR was heir to Oberth’s equipment. At least 3 other groups unknown to VfR were at work. F. A. Tsander in Moscow wrote imaginately on space flight. He first started with gasoline and gaseous air, then LOX. Italian group, Guigi Crocco financed reluctantly by the Italian General Staff. Luigi’s father was General G. A Crocco, funded it apparently. “An Italian father is comparable to a Jewish mother.” He used gasoline unsurprisingly but also Nitrogen Tetraoxide, N2O4. Storable at room temp <- Nobody found this out until 24 years later. Pedro A. Paulet, Peruvian chemical engineer, claimed to do experimental firings with N2O4 in 1895-97 in Paris. Presumably false. Someone would notice sound of AA guns in Paris. 200 pound motor firing 300 times a minute. GLUSHKO! Headed rocket group in Leningrad. Suggested suspensions of powder beryllium in oil or gasoline as fuels. First firings in 1930 used straight toluene and N2O4, independently arrived at that oxidizer. VfR was unaware of all other work, Oberth wanted to use methane as fuel, but hard to supply in Berlin. Gas / LOX instead. Johannes Winkler independently fired a LOX - LCH4 engine by end of 1930. Work led nowhere as methane had only slightly better performance to gasoline and was harder to handle. Freidrich Wilhelm Sander, pyrotechnicians, used something he merely called a carbon carrier. Suggested to be a light fuel oil or benzene with powdered carbon or lampblack. He also used nitric acid HNO3 as oxidizer. 5-20% dissolved nitrogen tetroxide as well. Esnault-Pelterie, aviation pioneer and aeronautical engineer, in 1931 worked first with gas ox then benzene + nitrogen tetroxide. Third experimenter to independently come up with this oxidizer. Pelterie and Glushko’s use of benzene and toluene is odd because they’re more expensive and and equal performance to gasoline. Pelterie tried tetranitromethane C(NO2)4 and prompted blew off four fingers. Glushko picked up from Sander and from 32 to 37 worked with nitric acid and kerosene. The combination is still used in the USSR. then he fully fired a kerosene and tetranitromethane engine. This work wasn’t followed up. Late 1931, Laus Riedel of VfR designed a motor with LOX and 60-40 ethyl alcohol and water. Performance below gasoline but far lower flame temp. This was VfR’s major contribution to propellant technology and lead directly to the A-4. Wernher von Braun started his PhD thesis on rocket combustion phenomena in Nov 1932 under Army sponsorship. By the end of 1933 the Gestapo moved in on the rest of VfR and it was dead. Eugen Sanger at the University of Vienna during 31 and 32 did many firings with liquid or gaseous oxygen and a light fuel oil. But he introduced Diethyl Zinc into the fuel line before the fuel to act as a “hypergolic starting slug.” He also made a long with of many possible fuels (hydrogen or pure carbon) combined with Lox or N2O5. He calculated performance but assumed 100% thermal efficiency which involves either an infinite chamber pressure or zero exhaust pressure firing into a vacuum. In reality it’s 50-60%. Like Tsander he pondered powdered aluminum added to fuel. Luigi Crocco got some funding to a monopropellant. This could be a single molecule like methyl nitrate CH3NO3 or a solution of benzene in N2O4. Methyl Nitrate: a molecule with one reducing (fuel) end and one oxidizing end. In 1932 he started tests with nitroglycerine (no less!) with addition of 30 percent methyl alcohol. He didn’t kill himself and extended the work to nitromethane CH3NO2. He ran out of money in 1935. Nothing much came of the investigation. Harry W. Bull, University of Syracuse, did some development with gaseous ox and gasoline. He tried to burn alcohol with 30% Hydrogen peroxide (highest strength available in the US). He created his own monoprop he called Atalene and was put in the hospital. Helmuth Walter, Chemical State Institute in Berlin, in 1934 and 1935 develop a mono prop with 80% hydrogen peroxide. Only recently made available. Luftwaffe could see uses for this and the work was not made public. Last strictly pre war work was at GALCIT (Cal tech). Feb 1934, Frank Malina started his PhD thesis project to develop a liquid sounding rocket. He also assembled a list of propellants with oxygen, “red fuming nitric acid”, and nitrogen tetroxide. He did testing indoors with nitrogen tetroxide and methanol. Got corrosive nitric acid all over all the expensive machinery. Labelled “suicide squad.” Kicked out doors, continued work until July 1 1939 when General Hap Arnold got the Army Air Corps to sponsor a project to develop JATO. From now on, all rocket work was military. # Peenemunde and JPL Von Braun’s work began at Kummersdorf-West, an artillery range near Berlin. The Reichswehr paid the freight and built up a rocket establishment around him. In 1937 he got his degree and was made technical director of the organization. They moved to Peenemunde and developed the A-4. Little prop work done on the A-4. They used the 70-30 alcohol-water mixture from VfR and Helmuth Walter’s 80 % hydrogen peroxide in a decomposition chamber with a small quantity of a solution of calcium permanganate in water to catalyze decomposition. The need for AA rockets slowly came up in Germany (they still believed Berlin would never be attacked by air). A-4 propellants can’t do this, so Walter’s hydrogen peroxide was the solution. 80-83% concentration was first achieved at Helmuth Walter’s Witte Werke at Kiel in ~1934. Peroxide is a good oxidizer but not a good monoprop. Walter worked out a fuel he called C-Stoff (peroxide was called T-Stoff. Hydrazine hydrate (N2H4 * H2O) ignited spontaneously with peroxide - he was likely the first to notice this. Fuel was 30% hydrazine hydrate, 57% methanol, 13% water, + thirty milligrams per liter of copper as potassium cuprocynide to act as ignition and combustion catalyst. Methanol was used because hydrazine hydrate was hard to come by and by the end of the war its percentage was dow to 15%. Bavarian Motor Works (BMW) was invited by the Luftwaffe to work on the monoprop and got a contract to develop a JATO unit. Zborowski (in charge of the program, Herman Mueller his second) believed 95% nitric acid was a better oxidizer and easier to get, I.G. Farben guaranteed unlimited quantities. They worked in 1940 on it and in 41 proved with a perfect 30 second firing at 3000 pounds force thrust. Even Eugen Sanger was convinced, he earlier thought LOX was the only oxidizer worth thinking about. They discovered aniline and turpentine ignited spontaneously with nitric acid. Noeggerath learned of this and joined BMW and created the name hypergol from greek roots. In spite of Charles de Gaulle’s efforts to keep English pure, this became a word in english as well as german. If hypergols pool up and don’t ignite immediately, you get an explosion, a hard start that likely destroys the engine and surrounding area. Germans put a 50ms upper limit. Zborowski named his propellants after plants. Nitric acid Salbei and fuels Tonka. Nice smells, Clark can hardly think of less appropriate names! The BMW testing was getting a chemistry textbook, soaking a rag in fuel, and seeing if it ignited spontaneously with a spray of nitric acid. They realized older rags burned more than newer rags, iron and copper as the metals or as salts catalyzed ignition. They added 6% hydrated ferric chloride to their 98% nitric acid and got a new oxidizer. 2000 fuels were tried at BMW where propellant development was directed by Hermann Hemesath. I.G. Farben started this testing too, with less imaginative names. Most of the fuels were aniline, many mixtures (hardly any pure oxidizer or fuel tests) were based on aniline family. Tertiary maines (triethyl amine) or Aromatic amines (xylidine, toluidine, aniline, etc.). BMW Tonka 250 comprised 57% raw xylidine and 43% triethylamine. It was used in the Taifun missile. Noeggerath added furfuryl alcohol to get Ergol-60 which he considered the best Hypergol, he reported “somewhat wistfully” that furfuryl alcohol was readily available ina the US and not Germany. The investigators filing patents as soon as they found mixtures they liked. Animosity for “stealing” patents. Around 42 to 43, I.G. Farben began a new series of fuels (not similar to the Tonka of Ergols) called the Visols, vinyl ethers. They were rapidly hypergolic with MS-10 (10% sulfuric acid and 90% nitric). This eliminating hard starts when engines were at low temperature. Dr. Heller patented a new mixture where iron catalyst was put in the fuel rather than in the oxidizer, since the latter contained sulfuric acid and iron sulfates are insoluble in nitric acid. The Wasserfall SAM used a Visol fuel. Many academic tests of additives that were too rare, too expensive, or too active. Many German missiles were designed to use peroxide. But the Walter Type XVII submarines threatened to use up the whole production. As nitric acid work was successful, the shift to it was inevitable. Many in Germany calculated theoretical performance the proper way. These equations took ages - could take a full day with a desk calculator says Clark. The characterized many fuels and oxidizers, wanted to do OF2 but they ever got enough of it. In 43 they were thinking of using chlorine trifluoride as it had just started production as an incendiary agent. Dr. Noegerath showed that if the A-4 instead used nitric acid and diesel fuel the range would appreciably increase because of higher density. The calculation had little impact aside from the planned A-10 baselining the propellant. This would have hilarious consequences in Russia says Clark. Tetranitromethane is storable, better performance than nitric acid, and high er density. But it melts at 14.1 C. and it explores as Pelterie discovered. German engineers wanted to to 64% TNM and 36% N2O4, doesn’t freeze above -30C. Goeggerath refused to let it in his lab. When false reports came the the US used it at large scale they heroically started synthesis and accumulated 8-10 tons, then found exactly nothing to do with it. Heinz Mueller of BMW tried powder aluminum or magnesium in diesel oil. Poor performance, chamber pressure of 50-100 psi instead of the aimed 300. But it did cover the pine trees in a nice shinty silvery coating, “very suitable for Christmas trees.” Monoprops were called Monergols. At Peenemunde Dr. Wahrmke tried dissolving alcohol in 80% H2O2, motor detonated and killed him. WM. Schmidding firm tried Myrol 80-20 methyl nitrate and methanol, similar to Crocco’s nitroglycerine-methanol. They managed to get firings and fair performance but too many explosions and never got reliable. At ARIB, some tried to make “Lithergols” (hybrids). Infecting peroxide or N2O into a porous carbon motor. Nitrous oxide decomposes exothermically into oxygen and nitrogen, trying to get the oxygen to react with the carbon for extra energy. When the team was captured, they said just a little more engineering was needed. It was 20 years before anyone god a hybrid to work. US development closely followed Germany. No A-4 and no high strength peroxide though. GALCIT was choosing an oxidizer. Oxygen and ozone weren’t storable so didn’t work for JATO. N2O4 was considered too poisonous. Red fuming nitric acid, RFNA, settled on, contains 6-7% N2O4. By 1938, JPL was established with Malins’ group and Von Karman at the head. A rather non prophetic statement was put in a report that the corrosive nature of the acid can be overcome by the use of corrosion-resistant materials. “Ha!” This report was an excellent survey of the field at the time. Many sophisticated and accurate performance calculations. The procedure was developed in Malina’s 1940 PhD thesis, essentially and inevitably the same as that developed in Germany. JPL ran into initiation and stable combustion issues. Metallic sodium suspended in the fuel help initiation a bit. And benzene was slightly but not enough better than gasoline. An accidental discovery on the other size od the nation helped them. Robert C. Traux did his own tests in off time with liquid propellants, served in the Navy and was ordered to the Engineeringing Experiment Station at annapolis with orders to develop a JATO. One of his staff, Ensign Stiff, while working on gas generators discovered that RNFA and aniline ignited upon constant. “Such discoveries are usually surprising, not to say disconcerting, and one wonders whether or not Ensign Stiff retained his eyebrows.” Frank Malina visited EES in Feb 1942, heard of the discovery and JPLs issues went away. They fires a 1000 lb thrust motor by the end of april. Traxu also used the combination. Professor Goddard himself developed a JATO for the Navy with Reaction Motors Inc. (RMI) firing in Sep 1942, fist Navy JATO. He used gasoline and LOX. RMI was founded in 1941 with veterans of the Americal Rocket Society. James Wyld, Lovell Lawrence, and John Shesta. Gas/LOX was too hot, so they mixed it with water through a metering value as it entered the chamber. Clark says this is somewhat less elegant than VfR’s solution. The RMI unit successful flew in the PBM (bomber?) in 1943. On one test on the Severn River the exhaust jet set the tail of the sea plane on fire, the test pilot rose (or sank) to the occasion and landed tail first. aniline-FRNA worked, but was an abomination. It’s more scarce than gasoline, in a dress shirt war everyone wanted it, it’s extremely poisonous, rapidly observed through the skin, and freezes at -6.2 C. Thx for the metric. Two interwound lines of research came out of this for the rest of the war. Reducing the freezing point and somehow making gasoline hypergolic with nitric acid. American Cyanamid was given a contract to look into additives that make gas hypergolic with HNO3. JPL worked on both research lines. JPL tried 88% nitric acid, 9.6% sulfuric, and 2.4% SO3. Similar to the German mixture, they believed this wouldn’t corrode stainless steel. At LPL (book typo) they tried o-toluidine, it was successfully fired by too scarce. Furfuryl alcohol comes from oat hulls and Quaker oats had tank cars of it. 20% of it in aniline reduced freezing point to 0F, -17.8C. The eutectic (lowest possible freezing point) was 51% aniline 49% furfuryl alcohol. -42C freezing point. Furfuryl alcohol is as hypergolic as aniline. JPL added aniline, diphenylamine, mixed xylindes, etc. to gasoline and tested ignition delay. None made gasoline rapidly hypergolic. 50% Xylindines would work, but no production in the US. White Fuming Nitric Acid (WFNA)? American Cyanamid tried to find additives and failed. But they would turpentine was magnificently hypergolic with mixed acid or RFNA. Good fuel on its own. Aerojet Engineering was founded in March 1942 as the manufacturing arm of JPL. All major JPL guys were cofounders. They had their own propellant research program, for many tears difficult to disentangle from JPLs. Aerojet tried crude N-ethyl aniline (monoethylaniline). Almost as rapidly hypergolic as aniline. -63C freezing/melting point. But just as poisonous and hard to come by. Monoethylaniline and mixed acid was used on the Aerojet JATO. Then on TMI’s motor for the Navy’s surface to air missile Lark. Surface-to surface Corporal used RFNA-aniline-furfuryl alcohol. Thee orgs worked on monoprops, JPL started in 194 or earlier and added small quantities of chromium trioxide & later other chromium compounds to the fuel. Aerojet found addition of 8% butyl alcohol worked. Bob Traux at EES almost killed when someone connected the wrong pipe to the right value. Save Altman, at JPL, tried a mixture of benzene and tetranitromethane which naturally detonated at once. The war was over and the German work came to light. # The Hunting of the Hypergol… German rocket scientists eager to surrender to the American interrogators that moved at the speed of the army or ahead of them. <- There must be completely remarkable stories from the fall of Germany and Americans going through the country. Von Braun’s crew deposited the Peenemunde archives in an abandoned mine. The US took everything, A-4 fully or partially assembled, the archives, Hydrazine Hydrate, high strength hydrogen peroxide, and the special aluminum tank cars to carry it. Alcohol Oxygen combination seemed good for long-range missiles, but the US had no immediate plans for these. Tonkas and Visols were no improvements on monoethylaniline or aniline-furfuryl alcohol mixtures. They knew missiles, guided or ballistic were the artillery of the future. Everyone put together lists of propellants. Lemmon of JPL presented to the Navy in spring 1945. Many more studies, North American Aviation, Reaction Motors, the Rand Corporation, Kellogg, etc. Each came to similar conclusions. Two types of studies, first is long-rang ballistic missiles or rocket to orbit artificial satellites. Everyone agreed LOX is an ideal oxidizer (fluorine is los density and a “holy terror to handle”). Conclusions on JATOs were less definite. Available oxidizers were nitric acid, hydrogen peroxide (when production starts in the US) and nitrogen tetroxide. N2O4 and 90% peroxide both froze at -11C. Hard to fight a war in siberia or the stratosphere. Maybe freezing point could be reduced or weirdies like CLF3. Most fuels were equivalent, so secondary characteristics had to be used to sort them. Hydrazine was an idea, anhydrous N2H2, not the german compound. Dave Horvit at RMI fired the hydrate with oxygen in 1950, no other tests Clark is aware of. Hydrazine hydrate looted form Germany was converted to the anhydrous base being being distributed for testing. Conversion process: reflux the hydrate of a barium oxide then distil over the anhydrous hydrazine under reduced pressure. It had a freezing point 1.5C above water. Haunting thoughts of pentaborane returned. Pouring aniline-RFNA produced dense clouds of highly poisonous NO2. Because it’s corrosive it would have to be handled in the field. Being splashed with aniline meant turning purple and blue and dying of cyanosis within minutes (decrease in blood oxygen). JPL suggests in 1946 WFNA and straight furfuryl alcohol. The alcohol is mostly safe, WFNA is just a corrosive as RFNA but didn’t give clouds of NO2. They tested the new mixture with a WAC Corporal motor and found no difference between 20-80 furfuryl alcohol, aniline mixture and RFNA. Fast ignition too. The WAC Corporal was conceived asa sounding rocket, little sister to 20,000 pound trust corporal when under development. Ancestor to Aerobee. RMI tests with 220-pound thrust Lark motor, it use mixed-acid and monoethylaniline, the reference propellant system. They tests 3 fuels and 3 oxidizers. 80 octane gasoline, furfuryl alcohol, and turpentine. Mixed acid, WFNA, and RFNA containeding 15% N2O4. First stage of Diamant burns turpentine and RFNA. Hypergolic starting slug for gasoline runs. Furfuryl and mixed acid was smoky and messy. Sulfuric acid and alcohol produced tars, cokes, and resins which clogged the motor. Turpentine gave hard start with RFNA and WFNA. Mixed acid (whatever that is) and turpentine + furfuryl alcohol and WFNA (RFNA performed better, but NO2 fumes) were preferred propellants. Furfuryl alcohol freezes at -31C, too high for comfort. Tons other other testing at JPL. Kellogg tried Ammonia and Methylamine (to reduce the vapour pressure of the ammonia) and showed 1.5% decaborane made ammonia hypergolic with WFNA. Bendix Corp in 1953 showed it was hypergolic by flowing ammonia over a lithium wire just upstream of the injector. Many strange propellants tried for seemingly no good reason, they had bad properties. At JPL and the Naval Air Rocket Test Station. Testing moved from rocket motors into the lab, didn’t want to blow up motors. Many labs independently developed their own testing apparatus’s, simplest was a dropped and an eye ball of course. Paul Terlizzi at NARTS wanted to take high-speed Schlieren movies of the ignition process. Clark has no clue why this would be useful. Many salvaged German submarine periscopes used for the setup. Clark was there for the first test?! Hydrazine and WFNA. There was a fire Dr. Milton Scheer warned (Uncle Milty). Paul ordered them to fire anyway. Of course an explosion and the optics all disintegrated, they found the lid to the chamber in the attic some weeks later. Great laughter. Many tests with ignition delay apaparatus’s. All results differed with slightly different methodology. Timing might have been from first spark or full thrust, or speed and efficiency with mixing. They agreed on the order of ignition speed of propellants, not milliseconds though. JPL and RMI started the ignition delay work earliest. JPL found the minimum ignition delay consisted of 60-40 alcohol-aniline. Close to the eutectic point (-43C). Corporal fuel changed from 20% FA (furfuryl alcohol?) to 50-50. They showed that Amines, particularly tertiary amines and unsaturated compounds were generally hypergolic. RMI work. UofT found that 30% tetraallyl silane makes gasoline hypergolic. Standard Oil of California was the first of the oil companies to get into rocket propellant research, Mike Pino measuring ignition delays. In 1954, Lou Rapp at RMI assembled results and found that the ignition of a hydrocarbon or an alcohol involved a double of triple bond. If none existed, it would have to be formed before ignition could take place. Pino found in 1949 that butyl mercaptan was rapidly hypergolic with mixed acid. Standard Oil of California had to remove this from their gasoline so had tons of it. Butyl Mercaptan was hypergolic with MA, high density, and not corrosive. But performance lower than a straight hydrocarbon. Horrible odor too. Clings to skin and clothes. They were exiled 200 yards from the main office and simultaneously started development of new propellant compounds. Chemists took over from the engineers, early 50s. Ethyl Mercaptal of Acetaldehyde and of Acetone were the results. Found an even worse smelling compounds. “This was too much, even for Pino and his unregenerate crew.” Buried it 200 yards away then in the SF bay. The services wanted to use gasoline. Then in the early 50s the Navy and Air Force changed to Jet Fuel, and they wanted the missiles to use Jet Fuel. Transition from piston to turbojets. Turbojets run on anything, coal dust to hydrogen. The services decided they wanted something easy to handle and available. Their jet fuel standard became derivatives of petroleum - a kerosene. JP-1 was the first jet fuel to be tried. Narrow cut (range of distillation temperatures), high paraffinic (many paraffins, high viscosity) kerosene. JP-2 was experimental. JP3 was remarkably liberal with too wide a cut so it boiled off at high altitudes. JP-4 is the standard for 707s to F-111s. JP-5 and -6 arrived since but haven’t replaced JP-4. RP-1 is a later story. No two barrels of JP-3 or -4 were alike. Jet engines mostly only care about the right number of BTU’s per pound, nitric acid rockets care about molecule shapes. It was acid-gasoline all over again. Coughing, choking, screaming motor. Someone thought sonic vibrations might promote combustion. Didn’t work. JP didn’t work, new Navy program “Rocket Fuels Derivable from Petroleum” in 1951. “Derivable”, you can derive RNA from anything organic if you try hard enough. Many more oil companies and NYU started experiments. Ignition timing of hypergolic additives. Shell and NYU focused on acetylenic compounds and Phillips Petroleum on amines. Standard Oil of Indian tried phosphorous derivatives. In the end had fuels with structures similar to nerve gases. When some of their alkylthio phosphates were fired at NARTS, two rocket technicians put in the hospital. Acetylenic work was because double and triple bonds aided hypergolic ignition. The fuel molecule only needed a weak point where oxidation might start. The parent molecule as acetylene and regarded hopefully by workers. Extra energy by the triple bond but low percentage of hydrogen. Pure liquid acetylene had the property of randomly exploding for seemingly no reason. In the 50s some unusual propulsion cycles were considered. The ramrocket put a rocket inside a ramjet. Get up to speed with the rocket, then the ramjet can kick in and hopefully the fuel of the rocket can be used in the ramjet. Eg. burn propyne or methyl acetylene in a monoprop rocket that exhausts methane and elementary carbon, then burn that. NYU tried tons of different compounds of the acetylenic family, while Shell concentrated on just two and hunted for additives. 2-methyl-1-buten-3-yne (iso-propenyl acetylene) and one other used by Shell. Note: Acetylenic family is a class of organic compounds with one or more carbon-carbon triple bonds. Simplest is Acetylene (C2H2). First additives were derivatives of phosphorous triamide (P(NH2)3). They became a major component of the mixture and someone explosive ignition occurred. 1,3,2-dioxaphospholane found to be one of the fastest hypergols ever. More work had to be done and now it’s forgotten. Between 51-55 Happell and Marsel at NYU prepared and characterized ~50 acetylenics: hydrocarbons, alcohols, ethers, amines, and nitriles. Dicyano-actylene was an unstable compounds with no hydrogen atoms at all. Useful as propellant but 3 triple bonds. Progressor Grosse of Temple University burned it with ozone in the lab and got a steady state temp of 6000 K. Temperature of the sun. Most acetylenics had poor storage properties and weren’t stable. Many explosions. The Air Reduction Co. found some that showed promise and started production of propyne, methylvinylacetylene, and another long name in 1955. RMI showed 10% methylvinylacetylene with JP-4 gave smooth combustion. But by the time this was done, acetylenics over overtaken by history. Developed to be abandoned. Homer Fox and Howard Bost ran the amine program at Phillips. Primary secondary ,and tertiary amines, saturated and unsaturated, allyl and propargyl amines, etc. monoamines diamines, triamines, tetraamines. They synthesized and characterized at least 40 aliphatic amines. And a few other function groups - OH groups and ether linkages. They concentrated on tertiary polyamines as it was known tertiary amines were generally hypergolic with nitric acid. Di to tri-tertiary amines might be more so. “Their guess turned out to be right, but one is reminder of E. T. Bell’s remark that the great vice of the Greeks was not sodomy but extrapolation.” Introducing a hydroxyl group produced an excessively viscous compound at low temperatures, “expectedly.” “Allyl-terminated amines were also rather viscous.” None were good as jet fuel additives but some were good as stand alone fuels. During testing at other labs, ignition was hypergolic and fast, combustion good and performance respectable. The saturated ones were stable to heat and suitable for regen cooling. However, they were also obsolete at birth. Hydrazine was high performance, good density, hypergolic with storage oxidizers but expensive. “Its price was high, but the nature of the chemical industry being what it was, and is, one could be confident that it would come down to a reasonable figure when anybody wanted it in quantity.” It was sensitive to catalytic decomposition, but the right tank materials and cleanliness fixed that. The services didn’t want to specify an exact temperature limit, but they finally settled on -54C. Navy for a bit wanted -100F, no clue how you’d fight a war at that temp. Everyone tried to get Hydrazine down to -54C without sacrificing the other properties. 8 agencies tried and failed, would’ve been easy enough to predict. 42% water-hydrazine got them there, but water degraded performance as it was just along for the ride and did nothing for the energetics of the system. V. I. Semishin in Russia determined part of the hydrazine-water phase diagram in 1938, then Americans, then British to finish it off. Ammonia was tried (it’s a fuel unlike water), but 61% needed, and that pulling boiling point to -25C. F. Fredericks in 1913 and 1923 reported on the hydrazine-ammonia phase diagram, then D. D. Thomas at JPL in 1948. 44% methanol had acceptable physical properties but performance was low. Don Armstrong of Aerojet tried lithium borohydride, got to eutectic freezing point of -49 C. However it was discovered that it decomposes on it’s own. Someone suggested LiBH4 as a freezing point depressant for hydrazine in 66 or 67. “This may indicate something beyond a profound and depressing ignorance of the history of one’s own technology, but I’m not exactly sure what.” T. L. Thompson of north American tried 15% HCN but it was so toxic it alarmed everybody. Around 1949-50 the LAR missile was being developed and 67% hydrazine and 33% ammonium thiocyanate could be lived with. However degraded performance and inconveniently high vapor pressure. Grok: High vapor pressure means more evaporation, safety hazards. Higher pressures in the tanks, values, and seals, loss of propellant over time, feed system issues with gas bubbles, temperature sensitivity. Dave Horvitz now of Metallectro Co. created HAM Juice, -50C. 9.1% hydrazine, 19.3 methylamine, 71.6% aniline. Wasn’t the answer they looked for, but got the Army to add 5% hydrazine to their aniline-furfuryl alcohol Corporal fuel, 3 years later upped to 7%. Hydrazine nitrate was one of the most thoroughly investigated. Naval Ordinance Laboratory and NARTS (Clark was there at the time). NOL found that 55% hydrazine and 45% hydrazine nitrate froze below 40C. Adding 13% water got them to -54C. However viscous at low temps, like to froth (bad for pumps), and liked to explode as NARTS learned. NARTS tried 41.5% hydrazine, 41.5% perchlorate, and 8.5% water, still liquid at -54C. However, loved to explode and blew a hole in the ceiling of their lab while Clark was there, almost had his head blown off dehydrating it in crystal form. Additives as freezing point depressants rapidly got nowhere. Then, the Rocket Branch of the Bureau of Aeronautics granted contracts to Metallectro and Aerojet to synthesize hydrazine derivatives and check suitability as rocket propellants. The three derivatives were monomethylhydrazine, symmetrical dimethyl hydrazine, and unsymmetrical dimehtyl hydrazine. Man we’re here! At NARTS Clark got a pound of monomethylhydrazine for $50 and recommended the eutectic form with 12% hydrazine and froze at -51C as the fuel to focus on. MHF-3 by RMI “a few years ago” is 86% MMH and 14% hydrazine “and there is nothing new under the sun.” HNO3 performance was 98% that os straight hydrazine. Not bad, but sensitive to catalytic decomposition. Symmetrical MMH melted at -8.9C. MMH at -52.4, UDMH at -57.2. Dave Horvitz at Metallectro found 60-40 UDMH-MMH froze at -80 C. Viscosity at that temp also allowed it to be used, hit the Navy’s magic -100 F number. Aston & colleagues at Penn the thermodynamic properties were determined (heat of formation, heat capacity, heat of vaporisation, etc.) for UDMH and MMH. UDMH was so thermally stable that it could be used for regenerative cooling. MMH was a little denser. UDMH less liable to catalytic decomposition. Either could be a additive for JP-4, but UDMH was more soluble & tolerate more water in the mixture. UDHM was a faster hypergol with nitric acid as it was not only a hydrazine but also a tertiary amine. Clark had his own MMH-hydrazine mix fired at NARTS in early 1954. UDHM was so successful in smoothing out combustion that it was the fuel for the Nike Ajax missile, 17% UDMH in JP-4. Final decision to concentrate UDMH was made on economic reasoning. Metallectro proposed using a modification of the classic Raschig process for hydrazine, by reacting chloramine with mono or dimethyl amine, according to consumer preference. In the bid they worked out a sliding scale of prices depending on size of order. Westvaco (very large company) proposed synthesis with nitrous acid reacting with dimethyl amine to form nitrosodimethylamine which is reduced into UDMH. Process can’t be used for MMH. They took a profit hit in the beginning and drastically underbid, got the order, Metallectro dropped out for good. First military specs for UDMH published in Sep 1955. Their marketing department was high on success and created the trade name DIMAZINE. Clark pitied their chemists. Mike Pino at California Research tried allyl amines, asymmetrical diallyl hydrazines. No improvement and sensitive to oxidation and polymerization. UDHM was the fuel to be burned with nitric acid or N2O4. Designers wanted that last possible second of performance. MMH was growing in popularity. Applications that didn’t required a low freezing point, hydrazine was used, Titan II lives ina steam-heated hole. “But Hydrazine has an unfortunate tendency to detonate if you try to use it as a regenerative coolant.” The fuel finally chosen was a 50-50 mix of hydrazine and UDMH. Aerojet was first and called it Aerozine 50, 50-50 by everyone else. MAF-5, MHF-5, Aerozine-50, Hydrazoid N, U-DETa. Tons of hydrazine-type fuels. For a vernier motor on Surveyor enough water was added to MMH to form monohydrate whose cooling properties were better than the anhydrous compounds. Ethylene dy-hydrazine synthesized by Dow in early 1962 had a freezing point of 12.8 C, but high density, so could be superior to DETA as a density additive. Now an array of available and high performance fuels was available to the designer. # …and Its Mate RFNA was hated for corrosion. When cooled a slimy, gelatinous, white precipitate formed at the bottom. Insoluble, could be good for fertilizer? Steel too, SS-347 drum. JPL published a study with a new drum and a few months old drum. HNO3 concentration declined from 92.6% to 73.6%. N2O4 from 6.3% to 11.77%. One drop of RFNA gave a scar Clark still had 15 years later. Very deadly to breathe it in. Protective suits for propellant handlers, hot and awkward and probably caused accidents themselves. Mixed acid is essentially WFNA when 10-17% added H2SO4. Less performance due to the sulfur atom and sulfuric acid is very stable, less reactive? He’d test new chemists by getting a henchman to drop a bit of a plastic glove in 100 cc of mixed acid, jet of flame. Test the nervous systems. The Navy and everyone was convinced mixed acid didn’t corrode stainless steel. Navy spent a ton on containers, $120 per drum from SS-347. Mixed acids corrodes stainless after an induction period and it’s goo is far worse than RFNAs. Within 2 years the Navy drums were scrapped. Water accelerated the corrosion and MA was hygroscopic, liked water. Drums increased in pressure over time too. WFNA was an alternative, but freezing point too high. Pure HNO3 freezes at -41.6C, & just as corrosive. Chemists thought pressure increase was due to the drums, tested with glass away from light and found pressure increased more than in drums. WFNA or Nitric acid was inherently unstable. Decomposed spontaneously. N2O4 was poisonous and -9.3C freezing. However, when no water its practically non corrosive to aluminum or stainless, even mild steel. It could be filled at the factory and never seen again by operators, but too high freezing point. 4 oxidizers, 4 sets of headaches. Freezing point of WFNA attacked by tons of researchers. Some directly, some with additives. Lewis Flight Propulsion Laboratory of NACA tested KNO3 solutions. Ignition was slow and frequently explosive. Exhaust also had potassium ions and free electrons, a plasma, making radar guidance impossible. Organic additives also didn’t work, some were explosive additives. Callery Chemical Co. tried perchloric acid and nitronium perchlorate, then nitromethane. It got down to -100F, but too sensitive and kiley to explode. Miko Pino tried sodium nitrite but it precipitated sodium nitrate. Also tried some water mixtures but were averse to this as he was always very conscious of the effect (pernicious) of water on ignition delay. Next Methane Sulfonic Acid, 16% in WFNA froze only at -59C (F?). No sludge and similar corrosivity to WFNA. More research bu by 1953 nobody cared. Many wanted to test ignition delay of WFNA, tried to get extremely dry acid. The General Chemical division of Allied Chemical and Dye Co. apparently had an especially effective acid still and got output with <1% water. Labs would store in cold places to slow decomposition. These tests proved water was the ignition delay issue and determined if a missile firing would be successful or explode. A field device to test WFNA for moisture was what they wanted. As simple and magic as possible. Dr. L. White of the Southern research Institute working for the Air Force wanted to shine an IR LED of the correct wave length to detect water. Studies by Ingold and Hughes, Dunning, and others in the 30s and 40s discovered an equilibrium in HNO3. So “species” water was always present, but they wanted “analytical” water. The relative between optical absorption and “analytical” water was not linear. White embarked on the calibration. At NARTS for the Navy, Clark was developing a method that tested on conductivity. Take 3 specimen and add varying amounts of water, take the ratios of conductance, then you can get the water and N2O4 content. You needed accurate acid analysis in the lab for calibration of the device, and current methods were at most accurate the a tenth of a percent. You had to determine the total acid content (HNO3 and N2O4), then determine N2O4, then determine water by the difference. Suppose you had 0.76% N2O4 and 99.2% =- 0.2% nitric acid, you get -0.16% to 0.24% water. All attempts were unsuccessful, he went above refining the direct method. Found many error sources. 5 gallons of 1.4 normal NaOH would have to be stirred for an hour to get uniform concentration. Would need to use a bubbler to unsure moisture in the air didn’t dilute the upper layer of NaOH. You have to be extremely precise with phenolphthalein. Needed extremely precise held at 25 C $75 to the taxpayer custom built burettes too. Some unnamed (Doc Harris!) people borrowed these and never returned them. The took almost a year and he got to 0.025% precision. He found a new simple was to get N2O4 that ran dry air over sulfuric acid and WFNA. It would get rid of ⅔ of the acid but it was 9 cents per pound. White published his paper at the end of 1951, Clark 9 months later. “And, then, naturally, everybody lost interest in WFNA.” 16 months later (after those 9 months) JPL made their own conductivity method. Dr. Harris at WADC (Burette stealing Harris) created an ingenious glass and teflon sample holder for RFNA that prevented loss of N2O4. Now it could be analyzed at accuracy equal to that possible with WFNA. Everyone promptly stopped using either mixed acid! N2O4 was better in many ways except freezing point. Many agencies tried fixing this with additives. Nitric oxide NO was an option. Wittorf in 1905 and Baume and Roberts in 1919 examined the phase behaviour of the mixture. It decreased freezing point but had higher vapour pressure. Other additives could decrease vapour pressure but at too high a cost. Eg. making it an explosive. Someone worked out the ternary phase diagram of nitrogen tetroxide, nitromethane, and TNM. Again, high explosives. “It appeared that about the only thing that could safely be dissolved in a nitrogen oxide was another nitrogen oxide.” “Cole, at JPL, had reported in 1948 that a mixture of 41.5 percent N 2 0 and the remainder N 2 0 4 had a freezing point of —51° and a boiling point of 33°. These figures so thoroughly contradicted the experience of everybody else that they are completely inexplicable.” NO is an extremely effective freezing point depressant for N2O4 because it combines, under pressure or at low temps, with it to form N2O3, so that the eutectic appears between pure nitrogen tetroxide and the composition corresponding to N2O3. NOTS guys showed that 25% NO gives -65F freezing and 30% gives -100F, but at 300 psi vapour pressure. Between 1950-54 NOTS and JPL guys investigated nitrogen tetroxide-nitric oxide in extreme depth. This bore fruit years later with Titan II. With mixed oxides of nitrogen (MON-25 or MON-30, number meaning percentage NO made a good performance (a high percentage of the theoretical performance) more difficult than neat nitrogen tetroxide. NO is very kinetically stable and slowed down combustion. It also had high vapour pressure so was turned away from for a few years. Though some space rockets use it “today.” Meticulous studies at Ohio State University and JPL solved decomposition and pressure buildup, the an unexpected breakthrough at NARTS solved corrosion. Now RFNA could be packaged into missiles. Monumental work of Prof C. K. Ingord and colleagues in a series of articles in 1950 laid out all the equilibria in N2O4. Then frank and Schirmer in Germany explained its decomposition. The result of the research is that in strong acid containing N2O4, 7 species are likely to exist HNO3, N2O4, NO2, H2O, NO2+, NO+, NO3-. And possible traces of H3O+ and H2NO3+. They found 2 more equilibria that cause gaseous O2 to emerger that is essentially insoluble in nitric acid. Pressure builds up and the acid turns red because of the NO2. The equilibrium oxygen pressure over 100% acid at zero ullage (unfilled volume) at 160 F is 70 atm. Bomb. D.M. Mason & crew at JPL and Kay & group at Ohio State published work in 1955 that laid out everything worth knowing about nitric acid. Thermodynamics, decomposition, ionetics, phase properties, transport properties, etc. A heroic achievement. RFNA now had a tolerable decomposition pressure (<100 psi) even at 160 F (71 C). JPL made a mixture called Stable Fuming Nitric Acid that contained 14% N2O4 and 2.5% H2O. SFNA froze at -65 F, Navy relaxed the -100 F requirement and the job was done. The solution to corrosion was simple. In spring 1951 at NARTS they (Clark & Eric Rau) found it when studying corrosion of 18-8 stainless steel, SS-347 specifically. Rau thought fluorine coating would decrease corrosion. General Chemical Co. division of Allied Chemical and Dye friend put some steel next to the HF line for a few days. When tested, corrosion was the same but delayed starting by 1-2 days. They found 0.5% HF in the acid and the coating became self-healing. “We reported this finding in our quarterly report, on 1 July, 1951, but NARTS was just two years old then, and apparently nobody bothered to read our reports.” Oct 10,11,12 Pentagon meeting devoted to tantric acid problems, 150 propellant people form industry, gov, and the services. On Oct 11 as they were to present in the evening they heard about a few papers where compounds with fluoride were tried as freezing point depressants, and they didn’t notice the corrosion effect. North America, JPL, and just about everyone jumped on HF and found it was more effective in aluminum than steel in gas and liquid states. But not universal, didn’t slow nickel or chromium and increase corrosion in tantalum by 2000x and titanium by 8000x. A technicians at Edwards was examining a set of titanium samples in RFNA when they detonated, sprayed him with acid and glass and filled the room with NO2. He died of asphyxiation without regaining consciousness. JPL studied it and found that intergranular corrosion produced a fine black powder of mainly metallic titanium. With wet nitric acid it was as sensitive as nitroglycerin. The driving reaction was the formation of TiO2. Despite the titanium debacle, they now had a usable nitric acid and the military specs for WFNA and RFNA were rewritten in 1954 by a group under Air Force sponsorship, Clark was there as a Navy representative. They threw out the terms WFNA and RFNA replaced with Nitric Acid, Type I to IV with 0%, 7%, 14%, 21% N2O4. I-A or III-A had 0.6% HF. Clark wanted to keep the HF trick secret, asked some CIA friends if it was know in the USSR and it wasnt and HF production was in trouble. He protested by the Air Force published it. The HF analysis method was too complicated for Clark so he devised a new method that worked overnight with a magnetic stirring rod and two beakers. Dave Mason of JPL found a faster method that used the colorimetric method and depended on the bleaching effect of fluoride ion on purple ferric salicylate. Type III-A became the standard and called Inhibited Red Fuming Nitric Acid. Only one important motor - second stages of Vanguard and Thor Able used type I-A with UDMH. Many didn’t know why it was called IRFNA, Clark saw one engineer fill a stainless tank with RFNA without any HF and wonder why his acid turned green. JPL tried “Maximum Density Nitric Acid” with 44% N2O4 and density of 1.63 (other numbers I’ve seen in this book are closer to 1). But once IRFNA was found, analysis dropped to zero. The industry started to trust IRFNA, sometimes a purchasing agent would analyze a drum, but it was mostly just filled with acid and on your way. For tactical missiles where freezing point matters, IRFNA type III-A is the oxidizer. 47,000 pound thrust Lance with UDMH or Bullpup which burns mixture of UDMH, DETA, and acetonitrile. Agena is 16,000 pounds thrust and uses IRFNA and UDMH. Overview of current uses: For strategic missiles, N2O4 is used. Titan II with 2x 215,000 pound thrust motors with fuel that is 5-50 hydrazine-UDMH mixture. Other nitrogen tetroxide motors are use in space from the 21,500-pound Apollo service engine with also uses 50-50, down to tiny one-pound thrusters for attitude control. Users and chemists, engineers are happy now. Performance, reliability, and you won’t have to go through that again. Afterword: Another symposium on liquid propellants was held at the Pentagon on May 23,24 in 1955. Lotta gloating while in 1951 symposium was all problems. Funny story of Air Force air-to-air NALAR missile being developed starting in 1950, and finally after 4 years of tons of experiments with seemingly everything under the sun, the found UDMH-IRFNA and it worked. The requirement was ready to fire after 5 years of storage, they tested after 12 and it worked. The engineers found HF to stop corrosion 2 years after the discovery, apparently they there was a lack of communication or they don’t read. They’d also been working with HF for a year. # Peroxide - Always a Bridesmaid Hydrogen peroxide was the oxidizer that never made it. Performance close to nitric acid. No toxic fumes, didn’t chew on skin like the acid. Didn’t corrode metals. But -0.5C freezing point. And unstable. It decomposes with heat and is an exothermic decomposition, so self accelerating. Many things could catalyze the decompositions. Most transition metals, organic compounds, dirt, OH ions. Stannates and phosphates with peroxide in trace quantities could stabilize by taking certain transition metal ions out of circulation, but posed issues when you wanted to decompose it catalytically. Very pure aluminum was best container, with very high cleanliness, worse than surgical. Even in perfectly clean conditions, decomposition still occurred slowly. You could put your ear to a tank and hear periodic glubs. Early in 1945 the British got a ton of 80-85% German peroxide. The same year they fired it in a motor with a calcium permanganate catalyst and furfural for fuel. They worked on it with many mostly hydrocarbon fuels. The rest of the German peroxide went to the US but had sodium stannate as a stabilizer and wasn’t too suitable for testing. Navy made a deal with Buffalo Electrochemical Co. which was just getting into production high high-strength peroxide. Becco got the German permanganate and diluted it down to 2-4% for mouth wase and hair bleach and gave the Navy 90% peroxide in return. JPL was first in testing from 1944-48. Tested from 87-100% with a variety of fuels, found that hydrazine was hypergolic with it, all other combinations needed a pyrotechnic igniter (kerosene, methanol, ethylene diamine, etc.). Hydrazine-Peroxide had a very low O/F ratio, 0.1-0.5 because of lots of oxygen in the fuel. MIT, GE, and Kellogg tried it with various concentrations. Kellogg tried a K3Cu(CN)4 catalyst with Hydrazine as the Germans had tried. NOTS was to develop a nontoxic propellant based on hydrogen peroxide and jet fuel, with fair low-temp behaviour. 9.5% ammonia form a eutectic which froze at -40C. This and all other mixtures were sensitive and violent explosives. The British pointed out in 1948 that Ammonium Nitrate was a good freezing point depressant and didn’t make it into a high explosive. The NOTS team tried 55% peroxide, 5% ammonium nitrate, 20% water. Frozen below -54 and fired with JP-1 in early 1951, but bad performance. Adding AN made the peroxide increase instability, making it likely to detonate in the injector and very bad to regen. A safer approach to ignition was found where you decompose most of the peroxide in another chamber and feed that into the combustion chamber. NARTS designed and fired a motor which incorporated the catalyst chamber in the main chamber. The Navy wanted peroxide for “super performance” for fighter jets. Leave the area very quickly. Hence the want for only an oxidizer that could use the existing jet fuel. Peroxide had traces of chlorides that made it corrosive to aluminum tanks. Chlorides come from all directions on the ocean. Some washington guys worried what would happen in a 10,000 gallon tank of peroxide had a wrench dropped into it. “[Washington man] wanted us at NARTS to build ourselves a 10,000-gallon tank, fill it up with 90 percent peroxide, and then drop into it — so help me God — one rat.” Carrier admirals are deadly afraid of fire. NARTS w/ Clark tested putting some peroxide in a big pan with JP-4 floated on top. It burned quietly, and foam put it our without fuss. Another test happened where as the layer of JP gets thinner, the peroxide underneath gets warmer and starts to boil and decompose, fuel is permeated with oxygen and peroxide vapor, then detonation. UDMH-acid tests weren’t as bad. They are so reactive they didn’t really get an opportunity to get close enough together to react horribly. A little water put things out. Acid-UDMH missiles got onto carriers. The british designed a JP-peroxide rocket plane. Peroxide as an oxidizer didn’t really get anywhere. # Halogens and Politics and Deep Space Peroxide, acid, or nitrogen tetroxide based storable oxidizers weren’t satisfactory, they wanted to try Fluorine and went looking for an easily decomposed fluorine compound. Most fluorine compounds are the ash of an element that’s been burned with fluorine, quite useless as propellants. OF2 was known but difficult to make and low boiling point that it was a cryogen. O2F2 unstable at room temp, ONH and O2NH low oiling points. Many more compounds that couldn’t be used. Halogen fluorines were looked at next. BrF3, IF5, and the sort. ClF3 was the best. Harshaw Chemical Co. spent tons of time and money in 1949-50 figuring out how to synthesize HClF6 and got nowhere but learned for OF2. The efflorescence of fluorine chemistry from the Manhattan project prompted more research into it. A bit was known by the time the rocket guys wanted it. Chlorine Trifluoride ClF3 or “CTF as the engineers insist on calling it,” books at 12 C so at moderate pressure can be kept liquid, melts at -76 C. Good 1.81 density at room temp. CLF3 is hypergolic with everything (from all fuels to test engineers), extremely reactive, no ignition delay has ever been measured. Asbestos, sand, and water it reacts explosively. Extremely toxic. It can be stored in steel, copper, of aluminum contains because of a thin film of insoluble metal fluoride. But it that comes off you need a good pair of running shoes. He tried to get the General Chemical Co. salesmen to tell him a story of an accident and they only did when he threatened to buy RFNA from Du Pont. A 1-ton steel cylinder of CTF (cooled with dry ice to make it easier to load) started cracking from the cold. It cracked and spilled it all into the factory where the ClF3 eat through 1 foot of concrete 3 feet of gravel. None killed but one man had a heart attack when running at full speed, he got 500 ft away. Town evacuated. Bell Aircraft fired it in spring of 1948 using hydrazine, other tests with NACA and North American, in 1951 NARTS burned it with both ammonia and hydrazine. Amazing ignition and performance, very close to theoretical. The reaction was very faster so it could be fired in a relatively small chamber. However, if any hardware was dirty it would ignite and be destroyed. Grease or oil. Gaskets and O-rings had to be metal too, no organics. Joints had to be polished and inspected many times. The plumbing priming process involved running water, nitrogen to dry it, ethylene trichloride to catch any traces of oil or crease, more nitrogen, then gaseous CTF was left there for a few hours, then liquid CTF. CTF chamber is near 4000 K. Engineers of course hated dealing with it. It was only recently after customer demanded better performance than IRFNA-UDMH that CTF was the subject of large scale, intensive testing. Bromine pentafluoride is very similar to ClF3 with slightly higher boiling point 40.5 C, however it never performed at close to theoretical and no one knows why. No fuels were satisfactory for use with CTF. Carbon based fuels are bad for a fluorine type oxidizer (see chapter on performance). Degrades performance and gives a smoky exhaust Tom Reinhardt of Bell, Stan Tannenbaym of RMI, and Clark of NARTS started trying to solve the same fluorine problem around the same time. Stan and Tom started with MMH and CH6N2. The Carbon atom needs to be dealth with, so they mixed 1 mole water with 1 mole MMH, you get the output CO and hydrogen burns to HCl and HF. Performance worse than hydrazine as energy is wasted in decomposition of water, but better than ammonia. Clark started with MMH and N2H5NO3 (from the work on hydrazine nitrate) as the oxygen carrier. He wanted to calculate performance so called Jack Gordon of RMI to get the heat of formation of MMH and hydrazine nitrate and he had the numbers on the top of his tongue. He determined it was 95% of the performance of straight hydrazine. They ran card gap tests (explode 50 grams of tetryl under a 1x3 inch tube of it, see how many 0.01 inch cellulose acetate discs it punches through, 0-100). It was shock insensitive, so they put it on a shelf for when the engineers would ended it. Called it Hydrazoid N. Call from Stan Tanenbaum to do some card tests. “Can’t tell you what, it’s proprietary” “fuck you stan” “substituted hydrazine with some oxidizing materials” “You’ve got 3 moles MMH and one hydrazine nitrate and” “who told you” “my spies are everywhere” “God forgive me, but I couldn’t resist the line. “Oh, my spies are everywhere,”” He called washington’s rocket branch and say RMI’s MHF-1 and NARTS Hyrdazoid were the same thing to kill stealing rumours early. Later in 1961 he tried N2H5ClO4. 4 moles MMH, and 4 straight hydrazine. 98% performance of hydrazine and higher density. He found a way to make it without ever isolating the dry salt of hydrazine perchlorate, which as you remember is to be avoided. The engineers didn’t like it because if it caught fire as a spill, Hydrazine perchlorate became more concentrated and it would violently detonate. He called this Hydrazoid P, Hydrazoid N would do the same thing. He wanted catalyze combustion so the detonation wouldn’t take place.Certain metal oxide and ions catalyze hydrazine decomposition, but he only wanted this to happen under combustion. The answer was to wrap the ion in a protective structure, he told “one of the gang” to make the acetylacetonate complex of every metal ion in the stock room. Nickel acetylacetonate did a good job. Nothing at room temp or storage, but half a percent speed up combustion in air of as monoprop under pressure. Only detonated one third of the time. “A pity, too. For the nickel complex gave the fuel a peculiarly beautiful purple color, and somehow I’d always wanted a purple propellant!” Detonation hazard is only for spills but not prepackaged missiles. Cl2O7 with endothermic heat of formation of 63.4 kcal/mole was the most powerful liquid oxidizer know in the early 50s. Had great theoretical performance with many fuels but loved to explode. Olin Mathieson tried 70 additives as others did to to desensitize or stabilise it, none were found. Perchloric acid seemed more promising with an exothermic heat of formation, so little tendency to decompose. But 100% perchloric acid, like nitric acid, is not 100%, it has equilibria. When a very sensitive oxide triggered the decomposition it created chlorine, oxygen, and H2O. Now he ruminated on that fact and thought about replacing the H-O with F. Spring 1954, Tom Reinharbt, chief engineer of NARTS, Dr. John Call, director of Pennsalt Chemicals and Clark were sitting talking about propellants. John tried to sell them on NF3, but boiling point of -129. Clark suggested his FClO3 and asked John to make it. It had already ben made and the results were just what Clark predicted and they had just hired the man who discovered it. Clark was ecstatic and the Perchloryl Fluoride program began. Some Germans in 1951 treated NaClO3 with fluorine and must have made it. Then in 1952, Englebrecht and Qtzwanger, in Austria, made Perchloryl Fluoride as well and knew they had made it. They made a cutting torch with it, very dangerous and Pennsalt Chemicals hired Englebrecht. In June BuAer authorized NARTS to investigate perchloryl fluoride and Pennsalt gave then 33 grams painstakingly made with Englebrecht’s technique. They looked for a better method and Dr. Barth-Wehrenalp came up with and patented a synthesis. KClO4 + (excess) HCO3F -> KHSO4 + FClO3. Pennsalt was characterizing it at the same time Clark was, and they were exchanging information and getting new information by the week. In 1955 they were ready for motor work and Pennsalt hand-carried 10 pounds of Perchloryl Fluoride at $540 per pound with the old process and the new one wasn’t ready yet. They did small motor tests with MMH and found performance to be close to ClF3 hypergolic but with some hard starts because of some violently explosive minor compounds. So they used a starting slug of RFNA. Barth-Wehrenalp later tried mixing in a small amount of chloryl fluoride and got hypergolic ignition. However, it dissolved the proactive metal fluoride coatings that had made storage of ClF3 simple. It wouldn’t set fire to you and wasn’t toxic. However it had low density (1.311 vs. 1.809 for CTF) and between 25 C and 71 C it expanded 20%, High coefficient of expansion to to 95 C critical temperature. Tanks would have to be oversized, so it lost. However, it is missile with all-halogen oxidizers like CTF so could be added to help them burn carbon-containing fuels which need oxygen. “This will probably be it’s future role.” Other labs were trying to find better storable oxidizers. Colburn and Kennedy at Rohm and Haas reacted nitrogen trifluoride with copper turnings (yea, turnings? Literally what it sounds like according to Grok) at 450 C and produced N2F4. They electrolyze molten ammonium bifluoride using graphite electrodes. Nickel electrodes wouldn’t work. It was called hydrazine tetrafluoride and not dinitrogen tetrafluoride because organic chemists tried to name inorganic compounds and inorganic chemists made a mass of naming organics! Rohm and Haas came up with an esoteric synthesis method. React NF3 with fluidized carbon in an easy to control reaction, but almost impossible to get rid of C2F6 byproduct. Du Pont reacted at 600 C in a nickel flow tube to form N2F4 and NOF. Another route is reacting urea in aqueous solution with gaseous fluorine to for F2NCONH2, hydrolyze it with hot sulfuric acid to liberate the HNF2, then oxidize the difluoramine to N2F4. Callery Chemical Co. did this with sodium hypochlorite in a strongly alkaline solution. Du Point and HNF2 are the processes used today. I love high pressure chemical manufacturing processes. Dinitrogentetrafluoride (book wrote it like that) was a high-energy oxidizer and Aerojet burned it with Hydrazine and pentaborane and measure 95-98% theoretical performance. 1.397 density. However, -71 C boiling point, so not storable. With Sputnik in 1957, space became a consideration. -150 C became the standard for propellant storable in space, though it was custom to adjust this for whatever you were selling. Eg. OF2 at -144.8 C could be called space storage. N2F4 might be useful for NACA if not for missiles. N2F4 react with water to form HF and various nitrogen oxides with nitric oxide to form purple F2NNO. Like N2O4 forms 2NO2, N2F4 forms 2NF2. Lotta these little things get in it. As Clark’s writing, N2F4 is better understood but probably not going to be used in military or big booster applications. Deep space is a possible application. A saturn orbiter would have to coast for years and oxygen even given a lot of empty space as insulation might be hard to keep and N2O4 would probably be frozen solid. No clue how to parse this story, but heres the quote at the end: “And the moral of this story is that it’s always worth trying an electrical discharge on your mixtures when you’re hunting for new compounds. You never know what will happen. Almost anything can.” They’re doing some spectroscopy to try and get F2NOF. N. Vanderhook shows up. Bill Fox’s group found that ONF3 could be synthesized by the photochemical fluorination of ONH and by the flame fluorination of NO with a fast quench - though this one is best for mass production. He chaired a propellant synthesis meeting. Saw Rocketdyne and Allied to report on ONF3. Swapped the schedule the put them back to back to get a fight, sadly didn’t. June 1966, symposium on fluorine chemistry at Ann Arbor. One paper by Professor Neil Bartlett of UBC! Bartlett had discovered OIF5 and xenon fluorides. Bill Fox quickly had his research declassified and he presented immediately after Bartlett, describing several methods of synthesis and nearly every interesting property of the compound. Bartlett shrugged it off and “well, back to the old vacuum rack.” “but the incident is something that should be noted by the ivory tower types who are convinced of the intellectual (and moral) superiority of “pure” undirected research to the applied and directed sort.” Nitrogen Oxidetrifluoride (among other names) boils at -87.5 and density at that temp is 1.547. Less chemically active that dinitrogentetrafluoride (dear god use spaces please) and hence easier to handle. Its potential as an oxidizer is similar to N2F4, deep space missions. Rocket motors for deep space generally have a comparatively low chamber pressure 150 psia or less (10 bar). Motors for sea-level use are around 1000 psia (~65 bar), “In a few years it will be probably 2500!” (~165 bar). Now it’s ~4500! Aerosol sprays are injected by their own vapor pressure!!! During a coast period space storables can be kept below boiling points then heated with a coil so their vapor pressure would be well above the low chamber pressure of the motor and it would itself be the infection pressure source. Nitryl fluoride FNO2 or others could be great for this. Aerojet during 1963 did lots of work on this with complete success. Best to choose a fuel and oxidizer with a similar boiling point and vapor pressure. Eg. ONF3 at -87.5 C and ethane at -88.6 C. RMI and JPL starting in ~1963 did work on diborane-OF2 space-storable propellant. Pratt and Whitney, Rocketdyne, TRW, with NASA contracts and NASA itself focused on OF2 and light hydrocarbons, methane, ethane, propane, 1-butane, and assorted mixtures. All hydrocarbons were good fuels but methane was by far the best as a coolant, transpiration or regenerative, and with best performance. OF2 - methane combo is extremely promising. It just took a while for Winkler’s fuel of 1930 to come into its own! Compound A is the last oxidizer he’ll tell the story of. It was a great achievement of many propellant chemists careers and illustrate admirably the nontechnical, but bureaucracy and personal obstacles. While Walter Naya was doing electrical discharge experiments in 1960-61 (made NF3) he occasionally got trace quantities of compounds with absorption bands at 13.7 and 14.3 microns (infra red) - compound A and B. He got a new job and Dr. Hans Bauer was put to it. Got enough A to do mass spectroscopy, found it was chlorine from the stopcocks. Right as he theorised it was ClF5, Rocketdyne’s contract was cancelled at their solid plant in Texas made a “security goof”. Around March 1962, Dr. Thompson got some company R&D money and got 3 months and 2 chemists and told Lawton (who put Bauer on this earlier) they could work on anything. They got enough A to approximate molecular weight. 127 vs. calculated 130.5. Lawton had a Air Force research program and decided in desperation to use that program. They didn’t allow work on interhalogens but he figured if he succeeded all would be forgiven. “In the old Royal Spanish Army there was a decoration awarded to a general who won a battle fought against orders. Of course, if he lost it, he was shot.” Pilipovitch was put on it and he put Dick Wilson on it and within a week he found the four reactions that outputted ClF5. Had to explain to the Air Force, lotta issues and wtf did u do with this program. It settled down and he got a new contract for this work. ClF5 is very similar to ClF3, but 20 seconds better performance. Boils at -13.6 C and density of 1.735. All existing CTF techniques could be used on it. Propellant community was extremely enthusiastic. Clark found what he called compound omega which was classified at the time of writing, but 9/71 a frenchman reported on it himself. It’s an ideal oxidizer space-storable for reactions with carbon containing fuels like UDMH. OClF3. “Lawton and company tried, and are presumably still trying to get it, and Dr. Sam Hashman and Joe Smith, of my own group, hunted for it for more than three years, without any luck, although they employed every known synthetic technique short of sacrificing a virgin to the moon. (A critical shortage of raw material held that one up.)” Some attempts at increasing performance of ClF5 by adding N2F4 but came to an abrupt end with the vapor pressure of the liquid mixture was alarmingly high. Compound B ended up being tungsten hexafluoride from the tungsten filament ub the mass spectrometer, “Even Lawton can’t win ‘em all!” # Performance Thrust in Newtons can be gotten by multiplying mass flow (kg/s) by velocity (m/s) to get kg*m/s^2 or N. Increase mass flow or velocity to increase thrust. Even Clark believes in Metric. “The performance of a propellant combination is simply the jet velocity it produces.” Momentum is the important thing thrust*time exerted. Power isn’t very useful, it’s the rate of which thermal or chemical energy is converted into kinetic energy in the exhaust stream in this context. Kinetic energy is Mc2 where M is the mass (relative to the rocket? What does that mean?) of the exhaust and c is velocity relative to the rocket. Power, or rate of conversion, is Mc2/2 where M is mass flow kg/s. Above we saw Mc = F, thrust. So we get Power = Fc/2. Saturn V has a thrust of 7.5 million pounds force. Not mass which is an important distinction. “I don’t remember the exact exhaust velocity of the Saturn engines, but it can’t be very far from 2500 meters per second.” One pound force is 4.448 Newtons. In Metric (MKS units) there’s no confusion between massa and force!. We multiply and get 41.7e9 Watts for the Saturn V. Mass flow is 15 ton/s. You can consider it as a chemical reactor, with a very impressive throughput. We want to find exhaust velocity given chamber pressure, proper expansion through a nozzle, etc. We can rearrange E = Mc2 for c = (2E/M)½ (where M is the mass of propellant, which is hopefully the mass of the exhaust gas). E however is not equal to thermal energy, H in the exhaust gas before it was expanded. C = (2H/M * η)½ where η (eta) is the efficiency of conversion of thermal to kinetic energy. Η depends on chamber pressure, exhaust pressure, and the nature of the exhaust gas as it exists unexpanded in the chamber and as it changes during expansion. You need to know the chemical composition of the gas in the chamber and stoichiometry is not enough for this. Two moles of hydrogen and one mole of oxygen do not come out as two moles of water. Because of high temperature you have a lot of dissociation and get 6 species in total. H, H2, O, O2, and OH + H2O obv. So we have 6 equations to solve. First equation states that the sum of all hydrogen species individually multiplied by the number of hydrogen atoms in it, all over the same sum of multiples of oxygen atoms, is a constant ratio you already know. Here, it’s 2. Second equation: the sum of partial pressures of all species equals chamber pressure you chose. The four other equations are equilibrium equations of the type (H)2 / (H2) = K1 The problem of the equilibrium equations gets exponentially worse as you get more different elements. “But you’re stuck with it (remember, I didn’t ask you to do this!)” You have all the equations before you, then you guess at chamber temperature and guess some of the partial pressures, then solve. Iterate until the chamber pressure equals your predetermined chamber pressure. Eventually all species are in equilibrium and you have the right ratio of hydrogen to oxygen and so on, and they add to the right chamber pressure. Equilibrium Constants: “Devoted and masochistic savants have spent years in determining and compiling these.” Man, this is why I need to learn more chemistry. Next, calculate the heat of formation for all species and compare that figure to the heat required to warm the combustion products up to the chosen chamber temperature (the same devoted savants got these numbers). Of course the numbers don’t add up and you need to guess another chamber pressure and repeat. Eventually the heat (enthalpy) balances, equilibria all agree, and chamber pressure adds up and you have the right elemental ratios. Next you have to decide to make a frozen or shifting equilibrium calculation. Either the composition and heat capacity remains unchanged as the gas cools and expands, or the equilibrium shift in accordance with the changing pressure and temperature. One assumes reactions rates are zero, other assumes infinite. For a conservative figure you use the frozen equilibrium calculation. Plug into this formula with many terms that were learned in Chem 154, Simplify to get the second one with the third formula. eta = For the shifting equilibrium calculation, you assume entropies are constant for all species. Guess a final temperature, sum the entropies and see if you’re anywhere close. Repeat. “And yet I know people who have been doing performance calculations for twenty years and are still apparently sane!” The equations were so tedious that they were cherished as fine gold (or platinum for shifting equilibrium calculations) and everyone demanded a shorter approximate method. Mollier charts plotted enthalpy versus entropy with isotherms and isobars cutting across the chart. These were standardized for certain reactions. Some were very general just applying to a defined mixture of carbon, oxygen, hydrogen, and nitrogen atoms, without specifying what propellants were involved. These were easy to use and gave faster results but didn’t apply to exact problems. “The Bureau of Mines, with its extensive experience with combustion phenomena, was a leader in this field.” 1949 MIT method could be use=d for partially any combination in the CHON system, but had a messy procedure for any exhaust pressure other than 14.7 or camber pressure other than 300 psia. Clark made some improvements and published the NARTS Method of Performance Calculation in 1955. These and graphical methods essentially involve interpolation. The other group of methods gave approximate results of a frozen equilibrium calculation, mostly based on the c = (2H/M * η)½ equation. Minor products ignored so temp was often too high, also a lotta guessing and experience helped with guesses. Tom Reinhardt’s 1947 methods involved curves of temp vs. enthalpy and Cp vs temp. 10 years later Clark improves on it with getting R/Cp ratio averaged over the whole temperature range to produce a nomograph for calculating eta. NQD-NARTS Quick and Dirty method accurate to 1%. Other approximate methods up to 1963, but the days of shorthand and complete hand calculation - thank God! - are over. Computers started in the 50s and were limited and hard to use. Punch Cards. “The programmer protested that he couldn’t handle that many elements, and Tom Reinhardt retorted, “The carbon and the oxygen will go to CO, and you just tell the little man who lives inside that box to treat it exactly like nitrogen.” End of problem.” They had frozen or shifting equilibrium programs, and made “kinetic” programs. They took into account heat transfer from solid to gas in the exhaust. These are impossible by hand, and difficult to get reliable kinetic data. “As the computer boys say, “Garbage in — garbage out.”” “And there is one disconcerting thing about working with a computer— it’s likely to talk back to you.” “[Wrong letter or comma] and the 360 comes to a screeching halt and prints out rude remarks, like “ILLEGAL FORMAT,” or “UNKNOWN PROBLEM,” or, if the man who wrote the program was really feeling nasty that morning, “WHAT’S THE MATTER STUPID? CAN’T YOU READ?”” They’ve always wanted to take axes to their precious abacuses. Early workers used exhaust velocity, but isp (exhaust velocity divided by standard acceleration of gravity) is used now. It has some ludicrous definitions like isp being the weight flow of propellant. During WW2 the Germans had an even sillier measure of performance, “specific propellant consumption” which was the reciprocal of ISP. so tiny figures like 0.00426 per second. Best to think of ISP as a unit expressed in 9.8 meters per second. Then you retain the concept of mass flow. “(When will the U.S. ever change over to MKS?!)” Reconsider this equation: We want to max H/M. Energy contributed by a molecule of exhaust products is the heat of formulation plus its sensible heat above absolute zero minus energy required to break down to their elements (of course at 25 C). The last term is generally smaller than the first (otherwise you wouldn’t get a good propellant), sometimes like Hydrazine it is negative. Then hydrazine breaks down into hydrogen and nitrogen you get 12 kilocalories as a free bonus. Max H min N. Low molecular weight. You want the efficiency term, , to be as close to 1 as possible. Exhaust pressure over Chamber pressure is of course less than one, so you try to get as high an exponent R/Cp as possible. Low Cp exhaust products are best. So, 1. A high heat of formation 2. A low molecular weight 3. A low Cp Generally, good H/M means bad R/Cp and vice versa and if both are good, chamber temp can get uncomfortably high. Considering specific exhaust products, N2 and C are useless. HCl, H2, and CO are fair (H2 has a heat content of 2.024 kilocalories per mole above absolute zero, so H/M at room temp is 1.0 Kcal/gm). B2O3, H2O, HF and others are solid. B2O3 and Al2O3 are excellent. (Yes. B2O3 repeated) For the R/Cp term, the diatomic gasses with R/Cp above 0.2 are excellent. HF, H2, CO, HCl, N2. Lot more mentioned here. You need to find a compromise propellant. Pure hydrogen is best since it has more heat energy per gram than anything else. Excellent R/Cp makes it possible to use most of the energy for propulsion. You want as much hydrogen in the mix as possible. It has to be a mix in a chemical rocket because you need a source of energy. Oxygen or Fluorine work. You want excess hydrogen to keep temps low, R/Cp high, and so c high. “Hydrogen is so light that a considerable excess of it won’t harm the H/M term appreciably, and you get the maximum performance, generally, when you use only enough oxygen or fluorine to burn perhaps half of your fuel.” If burning a hydrocarbon with Oxygen you’re working with the CHON system. Slightly rich combustion gets more CO and H2 into the mix and improves R/Cp. Halogen oxidizer with a storable fuel optimisations: Fluorine atmos equal hydrogen atoms. Any carbon in the combination, get oxygen to burn it to CO, so no solid carbon in exhaust. If solids or liquids in exhaust like BeO and Al2O3, then cram as much hydrogen as possible. Start with requirements from the engineering group and customer. Eg. STM with wide temp range, fixed dimensions, packaged at factory, no visible trail, and of course much higher performance than acid-UDMH. ClF3 (called Compound A for historical reasons) has the highest performance of any practical storable combustion and all exhaust products are diatomic (⅔ HF). But temp constraint. MHF-3 C0.81H5.62N2 leaves a trail of black smoke. “Also, his professional soul (it’s the only soul he has left after all these years in the business) is revolted by the thought of that free carbon and its effect on the R/C p term and what it will do to his performance.” Add oxygen to take the carbon out of the exhaust. Results: MHF-3, Oxidizer: 80% ‘A’ 20 PF. O/F 2.18, performance shifting at 1000/14.7 pounds is 306.6 seconds. 290 on test stand is reasonable. If O/F is too high it’s lean and performance drops, too low and carbon rich exhaust. Density 1.39, chamber temp 3160 K. Then argue with the engineers for a while. Funny bit of the chapter. # Lox and Flow and Cryogenics in General Viking sounding rocket burned LOX and ethyl alcohol, like the A-4. Many experimental vehicles in the 50s that I’d like to hear about. Many other these (including redstone) used the auxiliary power source of the A-4, hydrogen peroxide. X-1 use an RMI Lox-alcohol rocket moro. Other alcohols were used. JPL as early as 1946 tried methanol and isopropanol by North American. Winternitz at RMI was pressured into trying methylal because the boss had a friend with a lot of it, is only there was a use. Foolishness. NARTS did some studies for Princeton using LOX and pure USL type drinking alcohol. Lotta happy sailors. X-15 needed something more potent. Hydrazine had a tendency to explode when used regeneratively. 1949, program conceived, not enough Hydrazine around anywhere. Bob Traux of Navy and Winternitz of Reaction motors would develop the 50,000 pound thrust moor. Settled on Ammonia as a second best fuel. JPL had fired oxygen-ammonia but RMI rocket it out in the early 50s. Ammonia was very stable, so tough to burn and many rough running and combustion instability issues. Tried additives like methylamine and acetylene. 22% of the latter gave smooth combustion but dangerously unstable. Combustion issues solved by improved injector. Clark could hear the motor being fired 10 miles away and from the noise knew the process. Lou Rapp of RMI flew across the country to see the test flight. Someone asked him what he thought of it, mechanical monstrosity accident waiting to happen, that someone was Scott Crossfiel, the pilot. Thor and Jupiter were the first real IRBMs. LOX JP-4 with turbopumps. Very fuel rich to not melt turbine blades. JP-4 had better performance than alcohol, getting rid of peroxide simplified it. JP-4 burned all right, but in cooling channels it polymerized (high percentage of olefins) and stopped the flow, then the engine burned up. Soot, coke, and other deposits in the engine too. Hydrocarbon performance was required, but not alcohol. JP-4 specifications were sloppy to ensure a large supply at all times. I love American industry. They realized the missiles only needed to work once, so your fuel specifications can be as tight as you like. RP-1 specification was the result. January 1957. -40 C freezing, max olefin at 1%, aromatics at 5%. It was usually delivered better than specs, a kerosene in the C12 region, H/C ratio of 1.95-2.00. 41% normal and branches paraffins, 56% naphthene. 3% aromatics, no olefins. Rocketdyne was the autonomous division of North american for all the rocket work. Madoff and Silverman of Rocketdyne wanted a better fuel, tried dimethylcyclohexane. But by the time it was ready, Atlas and Titan 1 (first ICBMs) were designed around RP-1, Titan 2 used storables, and F-1 used LOX and RP-1. LOX and RP-1 never burn perfectly clean, that excess carbon is what makes the flame so bright. N2O2 and 50-50 like on Titan 2 is far clearer with visible shock diamonds. Oxygen motors ran very hot, a problem since the beginning. GE came up with the fix, 10% ethyl silicate in the fuel which deposits a layer of silicon dioxide for insulation to cut down heat flux. It’s continually ripped away and redeposited. 45% lower heat flux with more GE experiments like 1% GE silicone oil in isopropanol.Vanguard used such a heat barrier Winternitz at RMI used ethyl silicate in ethanol and methylal in 51 and with 5% ammonia cut down heat flux by 60%. Ignition of oxygen motors was hard since the A-4. Pyrotechnic ignitors had complications and poor reliability. Sanger used a starting slug of diethyl zinc. Bell Aerosystems in 1957 used an ampule of triethyl aluminum to start an oxygen-JP-4 motor. Later, 15% triethyl aluminium and 85% triethyl boron was ruptured in the fuel lines for startup. In the US hydrazine-LOX was rare, not in Russia. Only 2 rockets, Jupiter-C and Juno-1 use it. They ran uprated Redstone motors redesigned for Hydyne (Rocketdyne developed 60-40 UDMH diethylene triamine) instead of alcohol. Tsiolkovsky loved liquid hydrogen, but large tanks made it useless for missiles and only used after WW2. Only 3 orgs in the US could produce at a combined rate of 85 litres, 13 pounds, per hour. Uni of Chicago, California, Ohio State assuming 24/7 production. H. L. Johnson in 48 at Ohio State Research Foundation burned hydrolox 100 pound thrust motor. Next year Aerojet installed a 90-liter per hour continuous unit, raised total US capacity to 27 pounds an hour. J2’s fired 80 pounds H2 per second. Hydrogen is super-cryogenic. Boiling at 21 K. Quantum mechanics predict the hydrogen molecule H2 would appear in 2 states, ortho with both nuclei spinning in the same direction, and para, spinning opposite directions. In 1927 D. H. Dennison pointed out in the Proceeding of the Royal Society that the transition from ortho to para might take days. Layer itw as would that it did. And each mole of hydrogen that changed from ortho to para gave off 337 calories of heat. Only took 219 calories to vaporize one mole of Hydrogen (from 21 K?). The usual mix of 75% ortho 25% para would completely vaporized in a few days, all without any heat from outside. A catalyst was needed so this transition happened during cooling and liquefaction, and not during use to save headaches. P. L. Barrick at the University of Colorado and Bureau of Standard at Boulder came up with hydrated ferric oxide. Many others came later. By 1961, liquid hydrogen was a commercial product. Entirely new kinds of insulation were invented for the shipping of liquid hydrogen in tank cars. If oxygen got in liquid hydrogen, it’d freeze and become an explosive. And of course it burns invisibly. Frozen at 14K, you can get a hydrogen slush that can still be pumped as a homogenous liquid with higher density. R. F. Dwyer and colleagues working on this at the Linde division of Union Carbide. Development state at time of writing. 30,000 pound thrust Centaur and 200,000 pound thrust J-2 are the largest hydrogen-oxygen motors. 1,500,000 pound M-1 on the way. Shame Tsiolkovsky (OR ME) didn’t get to see it. 27 feet tall, 18 foot nozzle exit. 600 pounds LH2 per second, 1.5 tons Oxygen per second. Electrical ignition, no too hard. Pilot flame in a small chamber, then light the main combustion chamber. Some work to make it hypergolic with O3F2 (Ozone Fluoride) dark red, unstable liquid, good for a week at 77 K (lox boiling point). Recently found out that it’s actually a mix of O2F2 and O4F2. Clark says it’s not likely it’ll supplant electrical ignition. Nuclear rocket’s are the ultimate hydrogen motors. 2000 K, the through nozzle. Graphite-moderated enriched uranium realtor. Hydrogen working fluid. Hydrogen or Methane at 2000 K dissolve graphite, answer is to coat hydrogen flow passages with niobium carbide. Phoebus-1 motor at Jackass Flats (lovely name!) in Nevada 1966. 1100 megawatt (thermal) reactor, 55,000 pound thrust level, ISP 760, 850 expected soon. 2300 K chamber temp. Phoebus-2 is 250,000 pound thrust and 5000 mega watt power (thermal) reactor. 2x hoover dam and the size of an office desk. Liquid fluorine work started in 1947 at JPL. They made and liquefied fluorine on site (impressive) and fired it with gaseous hydrogen and liquid hydrogen in 1948. By spring 1950, with Hydrazine. Impressive given the state of the technology at the time, miraculous. Density was low (1.108) and the military didn’t want it as JPL worked for them at the time. Back to the beginning of the book and Dewar’s low density figure. Some didn’t believe it and 54 years later, Scott Kilter of Aerojet got 1.54. Hilarious, 1951. Some textbooks still with wrong values. 1971. Jesus this book is 54 years old. Many started looking into Hydrazine-Fluorine. No ignition problems, fluorine is hypergolic with almost all fuels they tried. Extremely toxic super-oxidizer that lights noble gasses. Metal fluoride is the inert coating it forms that allowed steel, copper, aluminum, etc. casings. The Allied Chemical Co. trucks have an inner fluorine tank surrounded by a jacket of liquid nitrogen to prevent evaporation or escape. Rocketdyne built Nomad, 12,000 pound motor fluorine-hydrazine for upper stages. Bell developed 35,000 pound Chariot for third stage of Titan III, fluorine and mix of MMH, water, hydrazine balance to burn to CO and HF. GE developed 75,000 pound X-430 fluorine-hydrogen motor. Some work on Flox and oxygen and fluorine are miscible and have similar boiling points. Rocketdyne burned it with a standard Atlas motor at 5% fluorine. Flox-5. With RP-1 at 1000 psi chamber pressure 14.7 exhaust, shifting equilibrium, optimum O/F. theoretical Ox-RP-1 ix 300s. Flox 30 gives 316, Flox 70 (balances to CO and HF) gives 343, pure fluorine drops to 318. Not likely on big boosters and HF is rough on the pad and people. More expensive my orders of magnitude too. But for deep space, hydrogen-fluorine, “It’s on its way.” Ozone is produced by the Welsbach process with an electrical glow discharge in a stream of oxygen. Deep blue liquid. Useful in water purification, manufactured commercially. When a mole decomposes to during during combustion you get 34 kilocalories of energy. Sanger was interested in the 30s. As toxic as fluorine. Unstable, explosive reversion to oxygen. REversion catalysed by water, chlorine, metal oxides, alkalis, and others. Pure Ozone is lethal, so work was done on mixtures with oxygen. From 1954-57 Forrestal Laboratories of Princeton University fired concentrations of ozone as high at 25% wuthering ethanol. Ozone boiling point is 161 vs. oxygen at 90 K. At shutdown in the lines you get a mixture, as it heats up you get gaseous oxygen. You get two liquid phases of the mixture, one 30% ozone on 75%. As more oxygen boils off, concentration of the 30% decreases and 75% increases until you have one solution, all 75% ozone. Very sensitive. After many post shutdown explosions, rigorous purging procedures were adopted. Flush with liquid oxygen, gaseous oxygen, or nitrogen. Some work was done to stop it from separating into two liquid phases, 1954-55, G. M. Platz of Armour Research Institute, figured out that 2.8% Freon 13, CClF3 prevents phase separation at 90K, but not 85K, so it can’t go below that or you risk an explosion. 77 K is the boiling point of nitrogen. Ozone fluorine mixture was considered (30% is ideal for RP-1). Never fired to Clark’s knowledge. Also a horrible concoction. Some thought Ozone explosions were initiated by organic peroxides, or that it was just it’s nature. Some still hold out hope. # What Ivan Was Doing When the Russians took Germany, they put all the chemists at Leuna of I.G. Farben! to work on propellant research. Not propellant men, but to the Russians a chemist was a chemist. ARPA did a similar thing years later. At first they determined properties, then in Oct 1946 they were sent to Russia, either State Institute of Applied Chemistry at Leningrad or Karpov Institute at Moscow. They were synthesising new propellants, hunting for hypergols, and trying to make gasoline hypergolic with nitric acid. They took a similar path to the US, Vinyl ethers like the Germans in WW2 and US in 1948. In 1948-49 they worked on mercaptans and organic sulfides, like Pino. Investigated every amine they could get, and tried mixed function compounds like vinyloxyethylamine. They usually burned with gasoline, a pyrolytic or high-aromatic type in hopes of a good hypergol. For a while the best fuel was German-developed tonka 250, mixed xylidines and triethylamine. Second state of SA-2 or Guideline (US designations) ASM used by North Vietnam uses that fuel with RFNA. The Soviets produced hydrazine hydrate (none captured from Germany) by 1948. Little interest in hydrazine or derivatives until 1955-56, when Soviet chemists learned of US success with UDMH. By this time all German chemists were sent home. Lack of interest could be due to lack of compatibility of hydrazine and copper, engineers liked to make motors from copper because of great heat-transfer properties. Russian climates discountages hydrazine. UDMH is now one of their standard propellants. Some work with German High-strength peroxide, in 1950 they made their own. Little interest except by the Navy when they took over all peroxide work (useful for torpedoes). Nitric acids used in the late 40s and early 50s were 98% WFNA containing 4% ferric chloride as an ignition catalyst. They had issues with corrosion as well. They tried some corrosion inhibitors 2 years before California Research tried them. “As a first approximation, the range of am missile is proportional to its boost velocity, squared.” Noeggerath related boost velocity to exhaust velocity and propellant density with an equation. Higher density matters more for eg. JATOs than ICBMs. A German “bethought himself of Noeggerath’s equation” and decided a V-2 with nitric acid and a very high-density fuel would have a long enough range to make him Hero of the Soviet Union. He wanted to find such a fuel 10% toluene, 50% dimethylaniline, 40% dibromoethane. 1.4 density. Bromies ruined ISP. His Russian bosses put him before a People’s Tribunal and fined 4000 rubles for “Misleading Soviet Science.” Got off easy. They tried 8% colloidal aluminum suspended in aluminum stearate in kerosene as a high-density fuel, but it froze at -6 C. Tried tetranitromethane as an oxidizer and blew up a lab. SS-1A (NATO designation) was a copy of the A-4 and ran 70% alcohol. LOX was available in large quantity using the highly efficiency and fast air liquefier designed by Peter Kapitza. SS-2 of 1954 and SS-3 used LOX and 92.5 alcohol. In 1954 the US published specs for nitric acid with HF, so next came the SS-1B “scud” burning kerosene and IRFNA. Presumably with a triethylamine starting slug. They used a high naphthenic type, similar to RP-1, for less coking when used for regen. Many crudes available in the USSr. 2 “rocket” grades of IRFNA, AK-20 and AK-27, percentages of N2O4. With Scud it was clear there were 2 major design groups in the USSR, Soviet high command keeping peace between them. Split projects were sometimes also done in the US, contract going to Lockheed followed by one to General Dynamics. One group loved liquid oxygen and designed the SS-6, SS-8, Ss-10. SS-6 was the R7 that lifted Gagarin. SS-10 burned hydrazine-UDMH, similar to our 50-50. Other group did storable oxidizers. IRFNA or N2O4. N2O4 on big ICBMs in steam heated silos. SS-9 ICBM (R36) uses N2O4 and probably 50-50 maybe MMH, similar to Titan II. SS-13 (RT-2) was a 3 stage solid similar to minuteman. Soviet Navy missiles use N2O4 with UDMH or 50-50 or solids. Chinese ballistic millies based on the SS-3 modified to burn IRFNA and kerosene. They tried boranes in 1949-50 but not much. 10% ozone was fired in East Germany in 1952. Not followed up on. A recent long review article on perchloryl fluoride in a Soviet chemical journal all sources were western. Such review articles in the USSR usually mean the start of a research program. No word of fluorine or liquid hydrogen, although it’d be surprising if they didn’t consider the latter in a space program. Classified because of N-1 dev with the RD-57 / RD-56? They use the standard Oxygen, N2O4, IRFNA, RP, UDMH and its mixes. “When he wants more thrust, Ivan doesn’t look for a fancy propellant with a higher specific impulse. He just builds himself a bigger rocket. Maybe he’s got something there.” # Exotics Exotics are expensive, got boron, and probably don’t work. Didn’t actually cost a billion. Boranes are compounds of Boron and hydrogen. Diborane B2H6, Pentaborane B5H9, decaborane B10H14. At room temp they are respectively gas, liquid, solid. Alfred stock identified most boranes from 1912-33 while H. I. Schlesinger starting in ~1930 contributed to dev of synthetic routes. Diborane and pentaborane ignite spontaneously in the atmosphere. They react with water to form hydrogen and boric acid. Repulsive odor and poisonous. Also expensive because synthesis is neither easy nor simple. Extremely high heat of combustion. 50% more than jet fuel. 1937 Parsons at JPL first considered decaborane. GE got a US Army Ordnance Corps contract (Project Hermes) to investigate boranes and develop large scale synthesis for air-breathing primarily jet engines. Paul Winternitz at RMI in 1947 did the first performance calculations on boranes. No computers and bad thermodynamic data. The likely inaccurate numbers looked encouraging. Diborane and LOX motor firings came next. Diborane was rare, 40 pounds in existence, ~50 pounds thrust and very short. An engineer told Clark he could feel the price of a Cadillac going down the tailpipe every time he pressed the button. Bad results, far lower than theoretical performance and B2O3 and elemental carbon deposits on throat ad diverging section of nozzle. Ordain and Rowe at NASA-Lewis fired the same combo in 1948 and got similar results. Hydrogen peroxide as oxidizer didn’t improve things. Glassy deposits and bright green flame were characteristic of borane firings. No good results for 12 years. Ordin in 1955 did diborane and fluorine and didn’t get any deposits - BF3 is a gas. But very hot combustion. Project Zip by BuAer of the Navy in 1952 continued from Hermes to find a borane fuel for bombers, before ICBMs increased range was a great thing. By 1956 the project had so many sub contracts and was a mess and split in 2. Originally Olin Mathieson Chemical Corporation and Callery Chemical Co. Tons of propulsion, chemical, academic organisation by the end of it. Callery got “Zip,” Olin Mathieson got “HEF.” Trade journals played up Up and “Super” fuels and many lost money on buying boron stocks. Evident that for desired physic properties similar to jet fuels, alkyl derives of the boranes are required. 3 of these went into production. HEF-2, propyl pentaborane. Callery’s HiCal-3 and Mthieson’s HED-3 mixes of mono-, di-, and triethyl decaborane, and HiCal-4 and HEF-4 mixtures of mono, di-, ti- and tetramethyl decaborane. The numbers signify intermediate states of synthesis. Pilot plants for both companies and handling and safety manuals published. “Never had one poor element been given such concentrated attention by so many chemists and chemical engineers.” The whole program stopped when better fuels weren’t required with ICBMs coming on, and the B2O3 which under 1800 C is either a solid or glassy very viscous liquid that isn’t good for turbine blades. Efforts to reduce the viscosity of the oxide failed, HEF’s and HiCal’s just couldn’t be used in engines. Plants put on stand-by then sold for junk. It wasn’t a complete loss and 50 years of boron research was done in 10. Dick Holzmann of ARPA is one of the reasons this research was made available and not just in contractor archives, he heckled and published Production of the Boranes and Related Research in 1967. Carboranes discovered by Murray Cohen of RMI in 1957. Useful as high-energy solids in the future. Now large stocks were available, and Aerojet starting in ~1959 worked with HEF-3 and pentaborane, burning with N2O4 or hydrogen peroxide. RMI got pentaborane-peroxide system good by 1964. Solid deposits weren’t a problem with larger motors. Don Rogillio and Edwards AFB in 1962-64 burned it with NF3 and N2F4 and of course got no deposits (F). Low density (0.618) and best oxidizers - peroxide and N2O4 - had bad freezing points. Tantric acid or halogen oxidizer (ClF3) lower performance. B2O3 in the exhaust made it very obvious. This expensive too. Diborane could be used in deep space. -92.5 C boiling, 0.433 density at boiling point. OF2 or ONF3 oxidizers good. 1959, it was under investigation by RMI and NASA-Lewis. Hard to design injectors and nozzles that’ll stand it. Combination is also very poisonous but hasn’t killed anybody - yet. BN system by Callery Chemical kept pentaborane in the picture. Boron nitride is a white crystalline solid with hexagonal structure similar to graphite. Note that carbon can be both a graphite or diamond structure, and BN can do this as well according to recent (1971) work. Heat of formation is 60 kilocalories and would be the energy source. For pentaborane-hydrazine combinations it has a theoretical ISP os 326 and chamber temp of 2000 K (1500 K below anything else around that performance). Air Force had tons of pentaborane from their Mathieson operation and no clue what to do with it. Practically free, and everyone leaped at it. Callery, NASA)Lewis, RMI, EAFB were some of the first. Most at ~100 pound thrust level. C* (cee star) is a measure of combustion efficiency. Deriver by multiplying the measured chamber pressure by area of throat of nozzle divided by mass flow. Output in meters per second. With theoretical C* you can get a measure of completeness of combustion. RMI tried Hydrazine/pentaborane, hypergolic but hard ignition. C* of 85-88%, bad. They got 75% of the 326 second theoretical. The reaction isn’t direct to BN and hydrogen. Some boron is exhausted as elemental boron, leftover nitrogen combines with hydrogen to form ammonia. This doesn’t help performance. Hydrazine is water solution while pentaborane is oil-solution, two didn’t want to mix. This led to some BN monoprop work. You just need a great injector. Jove, Jackson, and Haberman learned this at EAFB from 1959-61. They got thrust from 100 to 5000 pounds force and C* efficiency from 75% to 95%. 30 injector experiments, each more sophisticated and complicated than the last. Pentaborane is still poisonous and hypergolic with the atmosphere. Handling issues. If sprayed a fire with water, it goes out eventually and the unburned pentaborane is covered in a layer of solid boron oxide or boric acid. If the crust is broken, fire starts again. Issues decomposing it too. Huge safety manual, and a Rocketdyne tech (they worked closely with EAFB on it) got hospitalized because of pentaborane. Edwards is in the middle of the Mojave desert, they built a 30,000 pound thrust motor from 1961-63. It’s so secluded any anything is fine, they spilled a ton of liquid fluorine once to see what would happen, no one was really impacted. The BN tests were spectacular as you’re see dense white clouds of solid BN rising 2 miles into the sky. First attempt had 75% theoretical ISP. With a custom 6000 drilled orifice injector they hit the 300s number. ClF5 arrived just as BN succeeded, so it was obsolete at birth. ClF5 had similar performance, denser, easier to handler, simpler and cheaper motor, and invisible exhaust. Cheaper by an order. 5 years of work mostly unneeded. Aerojet “fairly recently” 1966-67 investigated BN for a ram rocket that exhausted hydrogen, BN, elemental boron, and ammonia - these would be burned with intake air for extra thrust. “Perhaps the Edwards people didn’t labor entirely in vain.” Borohydrides also didn’t make it. First type i alkali metal borohydrides, LiBH4, NaBH4, etc. Opmoc s;ats. White crystalline solids, NaBH4 is almost stable in water and handled easily, others similar. Lithium borohydride was tried as a freezing point depressant for hydrazine by Don Armstrong at Aerojet at early as 1948. RMI tried in 1958 with similar results. Back at Aerojet in 1965, Rosenberg found 4% borohydride decomposed in 200 days at 69 degrees C presumably. Aerojet fired sodium borohydride and liquid ammonia with oxygen in 1949, worse performance that hydrazine. Not followed up. Patrick McNamara at EAFB with hydrazine solution of the sodium salt with chlorine trifluoride in 1965, again worse than pure hydrazine. Since type included ammonium and hydrazinium borohydride. Can be made in situ in liquid ammonia or hydrazine. Aerojet fired a mixture, suspected it was unstable, and did nothing else. Third type is covalent compounds Al(BH4)3 and Be(BH4)2. Violently hypergolic with air and liquid at room temperature. Some tests of similar fuels with oxygen and nothing sufficiently encouraging. A decade later, ~1960 Dr. H. W. Schulz and J. N. Hogsett of Union Carbide started development of Hybalines. “And - something rare in the propellant business - they did it with company, not government, money.” They made a compound, used unstandard formation figures, it was fired at EAFB, 281s, and put to rest. F. C. Gunderloy of Rocketdyne discovered that certain linear polymers of beryllium hydride and dimethyl beryllium, with chains terminated with BH3 group were viscous liquids and worked on them for 405 years. Extremely poisonous, toxic. Beryllium was rare and expensive. “The development of these compounds would have been an admirable academic exercise well worth serval PhD’s in inorganic chemistry. As a propellent development program it can be classified only as an unfortunate waste of taxpayer’s money.” 2 things ahead for exotics: 1. Diborane will probably be useful in deep space work. 2. The pentaborane/hydrazine, BN system should be very good in ram-rocket and similar systems. “And the people who lost their shirts on boron stocks will have to discover a better way of getting rich from other people’s work. For them, my heart does not bleed.” # The Hopeful Monoprops “Monopropellants, unlike Gaul, are divided into two parts.” Low-energy: RCS or tank pressurization. Merguery and x-15 use hydrogen peroxide for ADC. High-energy: bipropellant competitors. Buffalo Electrochemical Co. (BECCO) figured out a silver screen coated with samarium oxide is better than calcium permanganate to catalyse hydrogen peroxide. “I’m not sure whether samarium was chosen as a result of a systematic investigation of all of the rare-earth metals, or because the investigator had some samarium nitrate in his stock room.” Rocket on Rotor concept: ~50 pound thrust motors on helicopter blades with centrifugal force pushing the propellant to the ends. ROR would be used to lift off very quickly, Clark saw some tests, it was cancelled and might’ve bene useful in Vietnam. 1952-57. That was one application of H2O2 as a low energy monopropellant. Torpedoes could tolerate the high freezing point (ocean). Decompose H2O2 into superheated steam and oxygen. Steam ran your turbine for propulsion. However, the oxygen would leave a wake, so BECCA added enough tetrahydrofuran or diethylene glycol to make the output purely water and CO2. CO2 dissolves in water, steam is no problem. They had to add water to make combustion cooler and less of an explosion hazard. Propyl nitrate was another tried monoprop. Investigated in 1949-50 by England’s Imperial Chemical Industries that claimed it was harmless and non-explosive. Ha! The usual suspects of research labs (plus Naval Underwater Ordnance Station, the old Torpedo Station at Newport) worked with it either as an auxiliary power source propedo propellant, or mixed with ethyl nitrate. Easy to start with a hot glow bar or slug of oxygen and a spark plug. Clean and smooth combustion. However, when a gas bubble is inside and compressed rapidly (eg. water hammer when a value closes), it violently detonates. “Sensitivity to adiabatic compression.” Ethylene oxide C2H4O was the next monoprop. It’s an important chemical intermediate, so cheapy and in supply. Just a spark plug for ignition and decomposes into methane and CO mostly. However, it deposits code in the reactor. Solve with either silver lining (flame temp is very low) or adding sulfur. It also likes to polymerize in storage, getting a gunk. Acetylenics (methyl acetylene and di-isopropenyl acetylene) were tried but too much coking and detonation. Hydrazine at JPL investigated by Louis Duff 1948-51. Decomposes either to hydrogen and nitrogen or to ammonia and nitrogen. Depending on chamber pressure, catalytic effects, stay-time in the camber, etc. Best stated by flowing Hydrazine through a catalyst bed into the combustion chamber. Iron, cobalt, and nickel oxides are good catalysts put on a refractory substrate. They get reduced down to the metals and take over the catalytic role after startup. However, restarted more than a few minutes after firing is impossible. Shell Development Company from 1962-64 developed an iridium metal catalyst. No one really likes it, easy to drown and get in complete decomposition. Iridium is also the rarest of the platinum metals and expensive. The major supplier of iridium is in the Soviet Union. A thermal catalyst is another approach. Impregnante bed with Iodine Pentoxide for initial startup, or iodic acid, then use a high heat capacity and insulated surface. However, if you wait too long it still doesn’t work. Post WW2, a lotta monoprop work happened in England. Very interested in monoprop as competitors to biprops. As early as 1945 they fired the German 80/20 mixture of methyl nitrate and methanol, but found they couldn’t live with it. Waltham Abbey people thought of Dithekites, developed during the war as liquid explosives. 1 mole nitrobenzene, 5 nitric acid, varying percentage water. Super poisonous and corrosive. They found that if you dilute it enough to not blow your head off, you get bad performance. 1947-78, investigated mixture of ammonium nitrate and fuel dissolved in water. AN-1, usually 26% Ammonium nitrate, 50% Methyl Ammonium nitrate, 3% Ammonium dichromate (combustion catalyst), 21% water. But bad performance. 2 lines of high energy monoprop development in the US. JPL and NORTs from 1948 and 1954 (chapter 3) investigated mixtures of hydrazine and hydrazine nitrate, very thorough. The mixture performed better than straight hydrazine, but it was very explosive. It could be a good liquid gun propellant. Higher energy per gram than smokeless ball powder, since its a liquid and not a grain. On and off research since the 1950s, not really getting anywhere. Commercial Solvents used their own money (unusual) while ignorant of existing research (usual) to make a series of propellants similar to hydrazine mixtures, but based on methyl amine, and adding ammonium nitrate or others. Safe, but low energy and performance. Other line of research was nitromethane. By 1945, EES, JPL, and Aerojet worked with it and discovered adding 8% butanol desensitized it from exploding your fingers off. JPL found the optimum injector and chamber design. From 1949-1953, J. D. Thackery at Aerojet did a study, very long study. Ignition was hard. Spark only worked with a stream of oxygen. Thermite type igniter was the only type that could be used, not pyrotechnic. They found a spray of liquid sodium-potassium alloy at startup reacted with the nitromethane and got it going, but not easy to handle. Aerojet tried additives to get a small chamber with stable and efficient combustion. Settled on Chromium acetylacetonate. Adding nitroethane and ethylene oxide to reduce freezing point and desensitise. Meanwhile, Fritz Zwicky found adding amines like aniline made it far more sensitive, patented it as an explosive. They settled on 79% nitromethane, 19% ethylene oxide, 2% chromium acetyl acetonate. Named it “Neofuel.” Canadian Defense Establishment in 1950 tried additive with WFNA to get better performance, but again it was too sensitive. 1954 seemed to be a deadend for high-energy monoprops with Neofuel. Tom Rice of the Naval Research Laboratory wanted to dissolve pyridine in WFNa knowing it was resistant to nitration so it’d probably dissolve to pyridinium nitrate and not the nitro compound and be stable. He tried burning a bit in a liquid strand burner and got a bit, but had no test stand for larger tests. Strand burner is a nitrogen pressured gadget which allows for burn rate tests in a narrow glass tube under high pressure (like a combustion chamber). Measure burning rate as a function of pressure. Paul Terlizzi, Clark’s boss, told him of Tom’s research and Clakr realized that any amine dissolved in an acid from it’s salt form would work. He did some performance calcs and found that trimethylamine would do better than pyridine. He wrote a letter to the Rocket Branch in BuAer for authorization. Early June 1954. He started getting 100 pounds of each salt (trimethylammonium nitrate and pyridinium nitrate) before approval. Made pyridinium nitrate by dissolving in water, neutralize with nitric acid, boil off water, crystallize. Once during boiling down it emitted NO2 fumes, had to be hosed down outside. His performance better than Tom’s did on Tom’s burner because Tom’s WFNA had more work. Named it Penelope because they had waited so long for something like this. Trimethylamine nitrate was easy except that the volatile trimethylamine sticks to skin and clothes and smells horrible. Roger Machinist had that job, unfortunately. In Sep 1954 they got authorization and were told to focus on the pyridine mixture. Made a larger batch and gave it to the hardware boys. Hurricane Hazel dropped a tree on Clark and his MG. Out of action some time. When he got back he heard about the firings. They tried a pyrotechnic igniter, didn’t work. Bert Abramson (in charge of test work) took and acetylene torch andheated the motor red hot, got a few seconds of combustion. Then put a bunch of lithium wire in the combustion chamber (50 pound thrust motor), Penelope pooled at the bottom, then reacted with the lithium wire. Blew up, and propagated back the fuel lines to the tank, luckily only a few pounds there, but the test cell was exploded. The only time he separates a compounds name, tri hexyl amine. He got every reasonable simple amine on the market to test. They made them into amine nitrate salts, then into propellants. Half a dozen flasks in the lab with a different nitrate in it, all bubbling away simultaneously. Each man owned a flask, they were eating lunch and one was frothing up and down, like a man about to sneeze. Yelled “hit the deck” all obliged and a whoosh went, no damage except ego. Remember he went 17 years without a major accident. Some nitrates couldn’t be made into propellants, but would heat up. Once the fire department was called to deal with one of these, diluting with water and dumped in a hurry. Some of these didn’t want to crystallize, so they evaporated them to dryness over a steam bath. Some were liquid salts. Monomethylammonium nitrate was a clear, viscous, slightly greenish liquid. He wanted to card gap test all of them. They had gotten an older destroyer gun turret for the card gap testing, contain the glass and get the witness plate after firing. Goof-off ensign supposedly in charge of this while John Szoke did most of the work. A trend emerged with sensitivity ni the card gap tests and structure. Longer chain = more sensitive. Eg. propylamine was more sensitive than ethyl amine. Tripropylamine more than dipropylamine more than monopropylamine. Some didn’t followed this, methylcyclohexyl. This didn’t make much sense, and he started figuring out this mass of apparently inexplicable numerical data by working out a card-gap sensitivity to a function, phi, “floppiness coefficient.” Long derivation that involved Log base 3, but those cancelled out. It’s a function of carbon chains, lengths, and degree of branching. Eventually he got he heat of activation or the explosion process and it was similar to the strength of molecular bonds. He down selected from 33 but taking 35 cards as an arbitrary sensitively limit, then with 10 remaining dropped some on freezing point or dry salt instability in storage. Final selection on thermal stability. Thermal stability tester: 10cc volume with pressure pickup and recorder and burst disc at 300 psi. Buildup pressure and watch. Constant temperature bath. Process: put sample in bath at 100 C, in a few minutes pressure rose to 100 psi, stayed for 15 hour,s then pressure increased at an accelerating rate until burst disk failed at 17 hours. Plotting the logarithm of the time-to-burt against the reciprocal of the absolutely temperature, they got a straight line that confirmed the derived values from the card gap work! They found primary amine mixtures were more stable than secondary more stable than tertiary. They’d started naming the mixtures with feminine names like hurricanes. Roger Machinist made diisopropylammonium nitrate and so had the right to name it, he’d been to the opera the night before. Isolde. The came up with a special ignitor. Had to be really hot. Mixed up powdered aluminum or magnesium, potassium nitrate, or perchlorate and epoxy cement, le tit harden in a polyethylene tube. With an electrically heated wire they got great results, brilliant white flame and clouds of white smoke. They did this just outside the door of the lab and started it up as safety guys strolled by. The Isolde would put out the fire, they needed to figure out a way in which it’d be its own power source. They just needed a usable starting slug, then it’d be firing. UDMH was too much complexity. Candidate materials were placed in a horizontal one- or two-inch diameter lass tube with propellant sprayed in. High-speed camera for results. They found that lithium hydride and rubber cement oddly did the trick. They wrapped it around a 6” wooden dowel that was screwed into the injector with a ⅛” pipe thread. LiH was protected from atmospheric moisture in a seal test tube. First successful run in January 1956 and by april, smooth-running and workable system. They found that anhydrous nitric acid was better than ordinary WFNA. Isolde-120-A, 120 for mixture ratio and A for anhydrous acid. They wrote their reports, and what to report! Best monopropellant ever fired by anyone anywhere, 95% theoretical performance. No fancy injector required either, they used 6 commercial available oil-burner spray nozzles for 75 cents each. Nov 1956, reports released. But by June all hell broke loose and everyone wanted in. First was RMI just next door in March 1956 with a Navy contract to develop “Superior Liquid Monopropellants.” Too many compounds to name tried by RMI (by this time with both Navy and Army monoprop programs). “Reading the names is enough to dampen a propellant man’s brow!” Joe Pisani phoned from RMI in late 1958 for some thermal stability testing of propargyl nitrate. 3 cc in the bomb (thermal stability testing device) done outdoors lowered into the bath. Some wait, then it exploded rather violently. Oil everywhere that in the freezing air turned to what looked like tar. Burst disc didn’t go fast enough. “And third (crescendo and fortissimo) you’ll have a couple of flunkies up here within fifteen minutes to clean up this ( — bleep — ) mess or I’ll be down there with a rusty hacksaw blade. ...” I specified the anatomical use to which the saw blade would be put. End of conversation.” Washington told RMI to forget propargyl foolishness and focus on N-F compounds. This story is a little later. Stna Tannenbaum at TMI tried mixtures of hopefully inert oxidizers and fuel. “Bathtub chemistry” but strong nerves required. In WW2 the French dropped bombs of N2O4 and benzene. The two were only mixed after deployment, very touchy. Oct 1957, an optimistic from Air Productions Inc. proposed liquid oxygen liquid methane high-energy monopropellant. Lmao. First rule of lox is never let it get in contact with a fuel. JPL showed later that you could get this to detonate just by shining a bright light. He wanted to set up a LOX plant alongside a natural gas well, tank up your ICBM on the spot and push the button. Bill Cuddy at Wyandotte investigated alkyl perchlorates, such as ethyl perchlorate C2H5ClO4, as monoprops. Cuddy heard the old reports that it was extremely explosive and you’d probably need an iron mask to work on it. Cuddy continued, they were easy to synthesize but they could never fire them in a motor as they’d always detonated before they could be poured into a propellant tank. They next worked on tetranitromethane. Got nowhere as everyone else had. They tried nitrobenzene with TNM, card-gap test would sensitivity was over 300 cards. Clark in his own work refused anything above 30 cards. “But about this time some of the people in the monoprop business, when accused of producing something which was insanely hazardous, would answer blithely, “Sure, I know it’s sensitive, but the engineers can design around it.” (The engineers took a dim view of this.)” They tried firing it in a micro-motor, sometimes for guests, and it always resulted in detonation. In late 1958 their work finally shifted to amine nitrates. Jack Gould at Stauffer did a lot of crazy shit with a “High Energy Monopropellants” contract. Tried dissolving NH3 in NH4. Both stable and could’ve been high performing but didn’t dissolve. Tried mixing pentaborane with nitro-ethyl nitrate. Exploded on contract. Lotta other stuff. Every quarterly report he’d list hypothetical compounds that’d make beautiful propellants if only you could make theme, like di-imide, H-N=N-H. Late 1958, Rocket Branch got fed up and put him on NF systems. Amine nitrate monopropellants were still in the picture all this time. GE tried making a self-pumping motor with Isodle. They blew it up, goes to show why you shouldn’t develop a new type of motor with an experimental propellant. Boat and Fox at Phillips Petroleum made some nitrate salts with bi-tertiary amines and dissolved the salts in nitric acid to make AN propellants of their own. Discovered these were not very thermally stable, as Clark did too. NARTS tried higher pressure motors (1000 instead of 300 psi chamber pressure) and regen with it. They’d need a water flush at shutdown or the propellant left in cooling passages of the still-hot motor would cook off and probably blow up. Clark looked into quaternary ammonium nitrates, hard to make so never tried before. HE made tetramethyl ammonium hydroxide into a nitrate salt and into a propellant. With the thermal stability tester it’s stand up to 160 C for more than a week while Isolde cooked off in 50 mins at 130 C. They couldn’t get commercially available tetramethyl ammonium nitrate, so they got chloride and used a lot of expensive silver nitrate to do the conversion. They later tried to reclaiming the silver to use it over and over. They got enough for card-gap tests and found it was 5 cards, so 2x more pressure was required to set it off than Islolde. They cheered and christened it “Tallulah” as it was impervious to shock. Hey Elon. Early 1957. McGonnigle remarked to Clark that straight chain hydrocarbons in N2O4 were more sensitive than branched chains. Clark wanted to generalize the Tallulah insights more broadly. He found that Tallulah with the fuel ion structure 1 got 8 cards, the second straight line structure gapped 58. A difference of 50 cards due to structure alone. He tried normal pentane (line) and neopentane (cross) with N2O4. He found the same trend, neopentane had a lower card count. The issues with Tallulah was -22 C freezing point, “Such a beautifully symmetrical ion can hardly be restrained from crystallizing.” They tried the ethyl trimethyl salt “Portia” (he says don’t ask) and diethyl dimethyl ammonium nitrate “Marguerite” (also don’t ask). Portia didn’t meet freezing point specs at lambda = 1.1, and at 1.2 crystallized poorly and hygroscopic. Marguerite met freezing point specs but crystallised very badly, and hydroscopic. Outside firms manufactured these as they had the high pressure equipment required for these quaternaries. John Galt at Pennsalt and dr. Phyllis Oja of Dow helped here by talking their pilot plants into making those and absorbing the financial lass. NARTS got it for cheap. Next tried trimethyl isopropyl ammonium nitrate and that was the best results so far on all metric (slightly less thermally stable than Tallulah). They called it Phyllis. Process was get enough for thermal stability and melting point testing, then car-gap work, then look for someone to manufacture enough for engine testing. In 1958 Bost and Fox of Phillips got a new Air Force contract for monoprop work and had tons of new equipment. Most new quaternary amine nitrates they tried weren’t sufficiently soluble with N2O4 and adding water made them lose too much energy. J. Neff of Hughes Tool (yes, Howard) early in 1958 with a Navy contract worked on boron-based nitric acid monoprop. 1.5 years never got to motor testing, but some interesting chemistry. “My deplorable habit of giving propellants fancy names was apparently catching!” Phyllis was the most promising AN at beginning of 1958. By end of year it was wired by NARTS and Spencer King of Hughes Tool. Most tested used a UDMH slug or NARTS’ custom ignitor. John Gall of Pennsalt in summer of 1958 sent two amine nitrates for evaluation. “There was some delay — one of my gang goofed and made the test propellants up in the wrong proportions, so we had to ask for another sample — but we finally managed to run off thermal stability tests.” Good initial results, he wanted to test different structures like the previous tests. They got the first one from Howard Boost’s “ethane” salt. The third was another larger sample from John Gall. The second was made Jefferson Chemical Co. when they made N,N’ dimethylpiperazine and they quaternized it. 1: 2 hours. 2: 3 minutes. 3: 3 days and then the test was cut off. They found that 3 didn’t detonate even at zero cards, froze at -5 C but they kept at it. They named it Cavea. Latin for cage. They tested with the usual routine of burning the salt in the calorimeter for heat of formation, measuring heat of solution of salt in the acid, density & viscosity as a function of temperature. Single ended ions like resulted in too high a freezing point. He shopped around for months to find someone that’d make a large sample of nitrates of either ion. It had horrible thermal stability. BuAer and NARTS wanted lots of the salt for motor testing. He could buy it at $70/pound in 10 pound lots. Houdry PRocess Co. made it by reacting methyl iodide (expensive) with triethylene diamine, then metathesizing with silver nitrate. He tried a new method using Jefferson Chemical’s N,N’ dimethylpiperazine and plugging in a two carbon bridge to get the intended result. Pictures! He got 95% yield on the first try. Next was finding a cheap way to convert from bromide to the nitrate. He needed to oxidize (nitric acid would do this) then force it up to the bromate. Some chemical equations here. They found if they added the salt to the acid too fast, they got a brick-red precipitate of Cavea tribromide, the salt of Br3-. They now had a cheap simple route to make Cavea salt that didn’t require and expensive reagents. By middle of Feb 1959, they learned that Howard Bost, working with di-quaternaries, had independently hit upon Cavea and decided it was the best propellant to work it. The two programs converged completely. At a NARTS symposium they all agreed (rare) that Cavea was the best path forward. “The nineteen guests comprised eighteen PhD’s and one drunken genius.” Dr. Wayne Barrett of W. R. Grace Co. methylated UDMH to get the ion you see below. “[He] so new and innocent in the propellant business that he didn’t even start to run when, as he mixed up the propyl salt with the acid, the stuff had started to warm up and give off NO2 fumes!” NARTS tested Barrett’s propellant, had it detonate in thermal stability testing at minute 14. Barretts continued and it detonating on him at minute 17 3 weeks after NARTS’ did. He called Houdry and Jefferson to get interest in manufacturing the Cavea salt, bidding for 100 pounds. Houdry: “This was early in 1959, remember, the cold war was on, everybody was excited over missiles and space, and apparently everybody was convinced— falsely — that there was a lot of money to be made in the rocket propellant business.” He told both that the other could get a cheaper price. Playing both ends against the middle. With in a month they both had preliminary samples for approval. They both got new ways to remove the bromide. Jefferson was at $15/pound, Houdry at $75 but argued they could get to $50. Wyandotte gave up on TNM momoprops and shifted to the AN family. They got came up with this: and it was called Cavea B. Freezing point of -54C. Cavea B was fired by NARTS, GE, Wyandotte, Hughes Tool, then JPL. 94% theoretical performance in a small chamber. Smooth combustion, better than the now called Cavea A. Lotta other attempts at new propellants that failed. Many such cases. B-N bipropellants had combustion problems, it was hypothesized combining them into the same propellant or same molecule would alleviate this. McElroy and Hough, at Callery Chemical Co. worked on what they called “Monocals.” Addition compound of decaborane and 2-3 muleules (book) dissolved in 7 molecules of hydrazine. Mixing MMH and decaborane was done in solution, or they’d explode when it warmed up. The propellants weren’t uniform from batch to batch and this wasn’t understood. Extremely viscous and insensitive. 50-thrust motor, 4 of 5 tests started or ended in detonation. H. F. Hawthorne’s group at Rohm and Hass tested “Dekazine.” It was “indecently sensitive” to adiabatic compression. Tests at 500 pound never at Hughes, never got above 75% theoretical performance or solve the detonation issues. Hawthorne of Rohm and Hass discovered you could react the bis (acetonitrile) (An) adduct of decaborane in benzene with triethylamine and get a nice structure. Very strong acid, almost as strong as sulfuric acid. Based on thistesting, Clark thought he could make a peroxide-based moonprop from (N2H5)2B10H10. Some of the ammonium slat of the perhydrodeaborate ion was made, put a few milligrams on a watch glass and dropped some concentratedH2O2 and sharpest detonation he ever heard. You couldn’t dissolve enough of it in hydrazine to get the number of B atoms equal to N. Lou Rapp at Aerojet thought you could substitute one of the “Hepcat” cations for the hydrazinium ions in the salt to solve this. Clark didn’t have contracts to worry about and the brass seldom paid any attention to what he was doing. So, he testing the idea because Lou Rapp. Clark’s tests went well, and at a monopropellant meeting in August, Lou Rapp was talking about their progress and Clark decided to get him in the eye and say NARTS already did it. They were friends and these opportunities don’t come up often. The Army took over NARTS and it became the Liquid Rocket Propulsion Laboratory of Picatinny Arsenal. The Army had no interest in BN monoprops, so the work stopped. Clark says right decision, low density and horribly expensive. It mostly served to show that a B-N monoprop could be made. First monoprop conference in November 1953. First under sponsorship of BuAer then American Rocket Society then Wright Air Development Center. All monopropellants are inherently unstable and can be detonated if you go about it the right way. The first purpose of the formation was to standardize monopropellant testing methods between all the labs, so way to compare results from different labs at the time. Card-gap test was the first adopted in July 1955. One day, Joe Herrickes of the Bureau of Mines tried for a whole day to figure out why two card-gap setups side by side turned up equal result with one monoprop and different with another. After swapping our everything, it was the aluminum vs. steel cups. Steel was standardized and published. The final report was composed by highly self-confident and howlingly articulate individuals, at least six of them considered himself a master of English prose style. Whew! Next they wanted to standardize the Drop-Weight “explosiveness” test. All existing apparatuses for solids disagreed, not to mention liquids. Bill Cuddy of Wyandotte in March 1957 modified Don Griffin of Olin Mathieson’s test, standardised at the OM drop weight tester. Don Griffin, a free soul, took a years vacation and spent it in the Hula-Hoop business. “He said it made more sense.” The device measured adiabatic sensitivity. The falling weight compressed a standardized bubble of air. Adiabatic heating would set it off. It needed a very solid foundation, six foot cube of concrete which rested on granite. Al Mead of Air Reduction in 1958 standardized the thermal stability tester. A small sample heated at constant rate, the temp at which it started to warm up faster than the heating bath was taken as the takeoff point. It really measured different things than NARTS’ setup, but it worked. Detonation velocity was another test standardized. It’s one thing to have a detonation in your motor, another to have it propagate back at 7000m/s into the propellant tank. The critical diameter is the diameter at which the detonation dies out. Varies with nature of propellant. Materials of line, temperature, and probably more. 1958-1962, lots of detonation testing at all the usual suspects. Some successful at finding ways to stop detonations. Designing detonation traps was empirical engineering art, not science. One design looks like the proportional symbol, as the detonation goes through and destroyed the lines, it kills it’s future pipe and dies off. Sometimes the cut would start up a new detonation. Not too reliable. Mike Walsh of Clark’s group invested a detonation trap for Cavea B. It’d move through 1 inch lines but not 0.25 inch. He got a 2 inch section, put something in the middle and drilled 16 0.25” holes, same throughput but stopped the detonation. With one 10,000 pound thrust test of Cavea B they didn’t have Mike’s trap and substituted a battery of 16 0.25-inch loop traps in the line. Never found out if it worked, the motor blew on startup and the fragments hit the tank and exploded it. 400 pounds of TNT equivalent, never recovered anything to test if their substitute worked. 2 foot concrete walls of the test cell went out and the roof came in. The motor (heavy solid copper) went 600 FEET DOWN RANGE?! 5 foot square of armour plate went into the woods and cut off a few trees at the food, smashed into a granite boulder, bounced and sliced a few tree tops, and finally landed 1400 feet from where it started. This was right before NARTS was to be “disestablished” by the Army. These issues with monoprops made them not want to be used in the field. What if you’re hit by shrapnel? Solution was to have two chambers in your tanks, one with fuel rich mixture, the other with the acid to dilute it. Have a can opener arrangement inside that mixes them a few seconds before ignition. The issues was the freezing point of the 2.4 lambda mixture was too high. They tried “Ixobel”, dimethyldiisopropyl ammonium nitrate. He called a chemical company in Newark and they said it was impossible to make, as Clark was staring at it in his lab. LRPL tried fuel rich Isobels (~2.4 lambda) as low energy monoprops for APU’s and so on, found they worked well. DAve Gardner in May 1958 was to find a low-energy monoprop for APUs for the Air Force. Wanted thermal stability up to 200-300 C. Extremely stable mixtures are hence low-energy. He got some very heat resistance propellants by the end. He later tried CH3CF(NO2)2. Performance wasn’t amazing, C-F bond is stronger but the materials appeared to be stable to just about everything. He called it Daphne. “But, alas, Daphne, too, was a woman, and could betray him. And took out most of the test cell when she did.” He incidentally made some high-density oxidizers by fluorinating potassium nitroformate KC(NO2)3. He insisted on putting Clark on as a coauthor, which he wasn’t, because he tried that reaction first with tetranitromethane. This oxidizer came just in time to be made obsolete by ClF5. Stranger and hairiest chapter in the propellant story started with two unrelated but almost simultaneous events. First, APRA in June 1958 started Project Principia, getting large chemical companies into developing improved solid propellants. “To get the benefit of their fresh thinking.” Old line of propellant companies complained. They may have good equipment, know about molecular orbital theory and pi bonds, but they don’t know propellants! Principia got underway regardless but the propellant guys turned out to the prophets. Second, W. D. Hiederhauser of Rohm and Hass at a propellant meeting late in September announced that N2F2 in a gas phase reaction would add across an olefinic bond and form a unique structure, I ain’t getting into it, that’s not useful. Soon there were 9 orgs especially from Principia that were doing this research into N-F chemistry. Most compounds were liquids, not solids, and ARPA in Nov 1960 rewrote the contracts to allow work on liquids. Most were sensitive and explosive. So spark sensitive that they couldn’t be poured. Tiny static charges formed when poured frequently enough to trigger detonations. Only way to fix this was the make a compound with a large molecule and small number of NF2 groups, which was bad for energy content. Another bad thing was too much carbon in the exhaust, the solution was to add oxygen to the compounds, but this make it more spark sensitive. Work on figuring out how to get the right molecule composition and structure. Walt Wharton of the Army Missile Command at Huntsville was determined to make an NF monopropellant work or prove it couldn’t be done. He used IBA, the IsoButylene Adduct of N2F4 made by Rohm and Haas. It had 293 seconds theoretical performance. Straight IBA was extremely sensitive on the drop tester. Sheraton discovered adding less than 1% N2O4 made this negligible. Then started burn rate studies on a liquid strand burner. Ignition problems, a hot wire wasn’t too reliable. He found the detonation propagation was less than 0.25 millimetres. His early work with a “T” motor, injector and nozzleless chamber with observation ports for a high-speed camera. Tried a slug of ClF3 for ignition and got a detonation. Antimony pentachloride - of all things - gave a smooth and reliable start. They were working with an “expendable motor” with no nozzle and low chamber pressure (2 atm). They were mixing N2O4 and IBA remotely right on the test stand, good thing it was remote because once a 150 cc mixture detonated during a run. 1962-63 they sent some up to LRPL for card-gap work. 10 cards normally and with 1% N2O4, but 96 in the optimal mixture ratio for max performance, 1 mole IBA to 1.5 N2O4. 2 people usually operate the card-gap apparatus, 3 is optimal. For this testing at LRPL they had 7. The old destroyer turret had gotten old so they added internal armour. With their first test, 9000 bats were disturbed and came flying out and chose one rocket mechanic that was at fault. Running around in an acid-proof safety garment with a shovel trying to beat away the bats. Lmao Some talk of it as a bipropellant, but Wharton and Connaughton fired straight IBA as a monoprop at the 250-pound thrust level and never got above 80% of its low theoretical specific impulse. NF monopropellant abandoned in 1964. Just as Wharton started his IBA work, A. W. Hawkins and R. W. Summers of Du Point had an idea to feed a computer all known bond energies as well as a program for calculating ISP an juggle the structural formulae until it came up with the structure of oa monoprop with ISP above 300 seconds. “It would then print this out and sit back, with its hands folded over its console, to await a Nobel prize.” “The Air Force has always had more money than sales resistance, and they bought a one-year program (probably for something in the order of a hundred or a hundred and fifty thousand dollars).” They got crazy results that would’ve definitely detonated and the program was cut after a year. Some companies realized that if you got enough NF2 groups around a carbon atom you’d get a good carbon-type oxidizer and not just a monoprop. Cyanamid and 3M tried it. All compounds were difficult to make and expensive. Calculated performance was high and sensitivity was high as well. Many more attempts but when the NF oxidizer was diluted enough to be safe, it lost its performance advantage. Some thought that an OF structure attached to carbon would be more stable than the NF2 structure. Not much came of it. NF programs didn’t lead to much practical liquid propellants. “The record of this chemistry is now being collected, to be embalmed safely in a definitive text, so that nobody will ever, ever, have to risk his neck doing it again.” The original goal of Project Principia was hit (solids containing NF2), but still a long way to go and probably not operational before 1980 or so. High energy and stability in monoprop was irreconcilable. Cavea B was the best and almost made it, but didn’t. # High Density and the Higher Foolishness Hybrid rockets were around since Oberth in 1929 for UFA and BMW in 1944-45. Many organizations after the war tried and failed. GE in 1952 for Project Hermes was typical. They tried a polyethylene fuel grain with hydrogen peroxide as the oxidizer. Extremely poor combustion. When they tried to throttle the oxidizer-fuel ratio varied madly and was nowhere near optimal. This is to be expected as oxidizer consumption depends upon the rate of injection while fuel consumption depends on the area of the fuel grain exposed. Tinkering on the shape of the fuel grain did little good. The engineers were guilty of starting engineering before doing research. Nobody knew how a solid fuel burned. Did it evaporate and then burn in the vapor phase? Solid-stat reaction? NOTS was the only org that kept at hybrids. 1959, revival as Lockheed got an Army contract. 1961, ARPA got into hybrids and by 1963 there were at least 7 programs going. Clark was amused by the same pattern every contractor followed. First, get a computer to calculate performance of every conceivable combination, publish a huge report, everyone had the same numbers. He wondered if the taxpayers had gotten their money’s worth. Fuel grains recommended were: - Lithium hydride plus a hydrocarbon (rubber) binder; - Lithium hydride plus lithium metal plus a binder; - Lithium hydride plus powdered aluminum plus a binder. And the oxidizers recommended (not necessarily in the same order) consisted of: - Chlorine trifluoride plus perchloryl fluoride; - The same two plus bromine pentafluoride; - Or, plus N2F4; - Or, finally, and a little further out, straight OF2. Rohm and Haas investigated a different type of hybrid, one that’d burn without the oxidizer. Grain was aluminium powder, ammonium perchlorate, and a plastisol binder. Plastisol contains nitrocellulose and nitroglycerin. The combustion products included lots of hydrogen and carbon monoxide, N2O4 was meant to react with these. Steve Tunkel at RMI investigated a solid oxidizer. UTC had a Navy contract to investigate the basic mechanism of hybrid combustion. Should’ve been done 10 years earlier, but it’s easier to get money for engineering than fundamental research. They used a slab of fuel with oxidizer passing over it in a transparent chamber. Either polyethylene or methyl methacrylate (Plexiglass) fuel, OF2 oxidizer (wasn’t unheard of in hybrid work). They found that oxidizer reacts with the fuel only in the vapor phase and that consumption rate of the fuel depend on heat transfer from the hot reacting gases. They found extra mixing volume was needed downstream for good mixing of fuel vapor and oxidizer, this got them to reasonable combustion efficiency. This dialed back the density advantage of solids over liquids. UTC built one hybrid motor that powered a “target drone.” Hybrids were not the answer to everything and their place limited. Arcogels were thought of by Atlantic Research Co. in 1956. Mixture of powdered ammonium perchlorate, aluminum, and nonvolatile liquid fuel and carrier like dibutyl phthalate. Consistency of toothpaste, so they used special burner tips to spread the pasty ribbon out to expose max burning area. Good combustion but the high density wasn’t good enough to deal with designing an injector. Phil Pomerantz of BuWeps wanted Clark to try dimethyl mercury. He said it was harmless and easy to synthesize. It was extremely toxic and he suffered mercury poisoning on two previous occasions and didn’t care to risk it again. He phoned a man at Eastman Kodak in Rochester to make 100 lb of dimethyl mercury and ship it to NARTS. Horrified gasp, it’d fog all the photographic film in Rochester, they would have nothing to do with it. Clark instead thought to spray liquid mercury into the chamber to get the extra density Phil wanted. Low Cp so it’s help with performance. tf? He used Cavea A so that if they ever got to motor work it’d be easier to handle 2 propellants than 3. Found that ISP declined a ton but density impulse (ISP * Prop density) got higher. Concepts of 27.5% tank volume being mercury. He wrote it up and called it “Ultra High Density Propellant Concept.” He expected someone from the Bureau to ask what he was on, insead Phil bought it. They were to test experimentally. They had to build a scrubber (long cylinder with water sprays) to ensure everyone around Morris country, New Jersey and the country didn’t get all that mercury in them. You wanted this for a missile, Phil? Right as they built the scrubber the Navy shut down “disestablished” NARTS and they were ordered to ship the setup to NOTS. “Saved by the bell!” Dean Couch and D. G. Nyberg of NOTS took it over and by March 1960 completed experiments. 250-pound thrust RFNA-UDMH motor in the middle of the desert so no scrubber. With 20 volume percent mercury they got a 40% increase in density impulse (he calculated 43%). Technically, a complete success. A more practical way to get a high-density system was a metallized fuel, one light metal suspended in it. Idea from 1929, BMW tried it in 1944 without noticeable success. Dave Horvitz at RMI from 10447-51 burned 10-20% suspension of powdered aluminum in gasoline with LOX. No spectacular success. Difficult to get good combustion efficiency as lots of the metal didn’t burn and viscosity differed with temp so hard to make an injector and it settled at the bottom of the tank. In 1953 Boeing considered suspending magnesium in jet fuel and burning it with WFNA. Never got anywhere. Navy safety issues brought back the idea. Someone came up with the idea that a gelled fuel for rockets could be used on carriers so that leaks could be easily contained. Thixotropic gel, left to itself sets to a stiff jelly, slows slowly as thought high viscosity, but with a large force (if shaken or under high pressure through an injector) resistance collapses. It’s also less of a fire hazard. Considerable recent work for airliners to prevent fire after a crash. 5% divided silica would make nitric acid would make it a thixotrope. Easy to make others thixotropes. Lower combustion efficiency and loading was annoying, but it worked. Finding a good agent for halogen oxidizers was hard. Aeroproejcts tried mixture of ClF3 and BrF5. Card-gap value of zero cards, strangely. He warned Bill Tarply and Dana McKinney that the system was inherently unstable. They didn’t listen and Fred Gaskins was working with it late in 1959 when it detonated, lost an eye and a hand and suffered fluorine burns. Testing ended. A few years later gelling could solve slosh in space vehicles. Slosh means unpredictable center of gravity change means loss of directional and attitude control loss. 1965, A. K. Beardell of RMI investigated diborane/OF2 for deep space work and wanted to gel OF2. Finely divided LiF worked, but several percent needed and performance appreciably degraded. R. H. Globus of Aerojet later found that bubbling gaseous ClF3 (5-6% additive) make it freeze instantly to microscopic crystals that acted as the gelling agent. “For some reason” ClF5 wouldn’t work. Gelling was thought of a way to get 50% metalized fuels without having the metal settle out. It was also found that with particles on the order of a micrometer, Van der Waals forces became important and it would tend to gel the mixture itself. The investigators around the country found that making a slurry was a black ary not a sience. “Accomplished reliably only with the air of witchcraft.” RMI created a slurry of boron in a hydrocarbon intended for a ram-rocket with chlorine trifluoride as the primary oxidizer. Rocketdyne as early as 1962 fired a aluminized hydrazine-hydrocarbon emulsion with N2O4. It failed too. NOL fired their “Notsgel” successfully, but it hasn’t found an application. Shelf life is an issue (~5 years), particularly with varying temperature - outdoors - or vibrations - shipping - the metal would settle out. Most gels and slurries were based on hydrazine or hydrazine mixtures. This meant impurities in missiles aluminum tanks would act as a catalyst to decompose the hydrazine. Some mechanism accelerates decomposition and there’s a gas phase issues that causes it to swell up or something. Can’t use gas to pressurize the fuel to expel it into the engine, the gas burrows through the fuel and most of it is left untouched. So a bladder is needed. Viscosity of the fuel and oxidizer vary with temperature at different rates, so mixture ratio differs. Unless temp above 2050 C melting point of aluminum oxide, it’ll be surrounded in a layer of solid or liquid alumina and refuse to burn to completion. N2O4 gives just about a high enough temp, nitric acid marginally so. Al2O3 exhaust would also be very conspicuous. At shutdown, the heat from the motor soaks into the injector, and gel in the injector holes sets into “something resembling reinforced concrete.” Without drilling, restarts are impossible. What a shitty propellant idea. Beryllium-loaded gels have some more fun problems. BeO exhaust is poisonous. Beryllium oxide melts at 4000 C. Rosenberg at Aerojet burned a beryllium-hydrazine slurry and got a C* of 70%, indicated none of the beryllium burned. N2O4 got that up to 85%. Aluminum hydride (AlH3) was always prepared solvated with ether which couldn’t be removed without decomposing the hydride. In late 1959, Dow Chemical and Metal Hydrides devised a method to make it without the ether. It was later not stable enough so it was abandoned. Beryllium hydride (BeH2) was known since 1950 but in an impulse state. In 1952, G. E. Coastes and I. Glocking of Ethyl Corporation managed to prepare it to a 90% pure state. It was nicknamed Beane (pronounced beany) for security. Later it was found heating it improved stability, then they got Baked Beane. One congressman apparently wanted to show his knowledge to a reporter and leaked everything he know. Liquids guys wanted to know if BeH2 could be used as a gel. Texaco, Aerojet, and Rocketdyne investigated it in MMH gels between 1963-1967. Aerojet claimed it was stable, Rocketdyne’s gel with straight hydrazine was not. With liquids that don’t have active hydrogens, it’s better. Grelecki at RMI in 1966 made a 55% slurry of BeH2 in dodecane and with hydrogen peroxide got high C* efficiency. Toxic exhaust, no more work. Heterogenous monoprop idea, solid fuel slurried or gelled in a liquid oxidizer. Midwest Research Institute came up with the first one in 1958, powdered polyethylene suspended in RFNA. 120 card sensitivity and thermally unstable. RMI did something similar at it was thermally unstable and scrapped. “About all it proves is the willingness of rocket people to try anything, no matter how implausible, if they can con NASA or one of the services into paying for it.” Tribrid “an etymological monstrosity, if there ever was one!”, sometimes called tripropellant. Early 60s calculations showed that for space use, 2 propellant systems had ISPs better than anything else by a large margin. First was Be-O-H system, burning beryllium to BeO. with hydrogen as the working fluid. Considerable interest in 1963 or so. Atlantic research approaches it was an extension of the hybrid system. Powdered beryllium made into a soldi grain with a hydrocarbon binder, burned like in a hybrid with oxygen, then hydrogen was added lower in the chamber downstream of the grain. Scrubber was needed to take the BeO out of the exhaust stream. They fired it and got poor combustion efficiency. G. M. Beighley at Aerojet tried the usually bi-prop arrangement. LH2 and slurry of powdered beryllium metal in LOX. Reported in 1966, never got >70% combustion efficiency and plagued by “burnbacks” of BeO2 slurry through in injector. “It’s really surprising that he didn’t manage to kill himself.” Combustion difficulties, toxicity of BeO, and price, not much point in continuing the Be-H-O system. Li-F-H system looked more promised. Li melting point is 179 C so it could be a full liquid tripropellant system. Bill Tarpley and Dana McKinney of Technidune (Aeroprojects renamed) reported gelling liquid hydrogen with lithium and lithium borohydride. Got good gels and discrete evaporation rate of hydrogen by a factor of 2 or 3. Rocketdyne went with a different approach, fired liquid lithium, liquid fluorine, and gaseous hydrogen. You have to keep liquid lithium hot or it’ll freeze in the propellant lines. Also keep it from the atmosphere. It’s also highly corrosive to most metals and anything you want to use for gaskets and sealing materials (it even attacks Teflon with enthusiasm.” The Rocketdyne crew overcame this and fired at 500 psi chamber pressure with high expansion nozzle (exit/throat area = 60) designed for space work. High surface tension of liquid lithium made injector design for droplets small enough to completely burn before going out the nozzle hard. Just lithium and fluorine got 458 SECONDS!!!!!! With 30% of mass flow hydrogen they GOT 542 SECONDS!!! Chamber temp of only 2200 K!!!! # What Happens Next Absolute chemical rocket performance limit seems like 600 seconds. Using free radicals or unstable species as propellants could get around this, use the energy of reversion to stable state for propulsion. Eg. when two hydrogen atoms form H2 you get 100 kilocalories of energy per gram? 50-50 by weight monatomic hydrogen and ordinary hydrogen could get to 1000-1100 seconds, if you could keep them apart and from reverting immediately in a catastrophic manner. No one has any clue on how to do this. Free radicals aren’t worth writing about. Nuclear is the only other way, “which, fortunately, works and is well on the way toward operational status.” Ionic and other electrical thrusters good for low-thrust applications, but Clark isn’t good enough to write about these. Guesses at which liquid propellants are going to be used possibly for the rest of the century: Short-range tactical missiles, ~500km range, RFNA-UDMH and gradually shifting to something like ClF5 and a hydrazine-type fuel. Monoprops unlikely for main propulsion, and no gels or slurries. Long-range strategic missiles, the Titan II combination, N2O4 and hydrazine mixture. “and if somebody wants to put a bigger warhead on the brute — I can’t see why — it would be a lot simpler just to build a bigger Titan than to develop a new propellant system.” First-stage space boosters: LOX and RP-1. They work and they’re cheap. Later there may be a shift to LH2 (shuttle! D4H!) but appears unlikely. Reusable booster won’t change it, but first stage ram-rocket boosters mean all bets are off. Hydrolox for upper stages, J-2 is very satisfactory and will probably be used for a long time. Later, as more energy is needed on the final stage, hydrogen-fluorine or hydrogen-lithium-fluorine or nuclear. Lunar landers, services modules, etc. could use N2O4 and a hydrazine fuel would remain useful for a long time. Deep space probes, methane,ethane, diborane for fuels (maybe propane) and oxidizers like OF2, maybe ONF3 and NO2F. ClO3F could be useful at Jupiter. No place for beryllium or N2F4 or NF3. ClF5 will probably displace ClF3 entirely. Pentaborane and decaborane and derivatives will fall into obscurity. Hydrogen peroxide will continue as a monoprop for attitude control and low-thrust applications. Probably not an oxidizer for main propulsion. “There appears to be little left to do in liquid propellant chemistry, and very few important developments to be anticipated. In short, we propellant chemists have worked ourselves out of a job. The heroic age is over.” “But it was great fun while it lasted.” |
| --- | --- |