# Amateur HAM Radio Operator Exam Notes

Christopher Kalitin

Resources:

https://hamstudy.org/canadaBasic2014

https://uwarc.uwaterloo.ca/assets/basic-essentials-main.pdf

https://quizlet.com/19575263/canadian-basic-qualification-question-bank-for-amateur-radio-operator-b001-flash-cards/

https://apc-cap.ic.gc.ca/datafiles/amateur_basic_questions_en.PDF

Two wires held apart by insulating rod is also known as an “open wire line” or “ladder line.” Can be used as an antenna transmission line.

“CQ” on voice means you’re looking for a response. Say 3 times followed by your call sign 3 times.

For your call sign, use the NATO phonetic alphabet.

Selective Fading:

Parts of a wave arriving with difference in phases. Large bandwidths are more susceptible to this.

Bandwidth is the portion of the RF spectrum that you can use (spectrum of Hertz). A large bandwidth would allow for frequencies than a small bandwidth. Eg. 18kHz-22kHz.

Amateur radio communication is allowed in cases of emergencies unless it is specifically prohibited. Mostly allowed always.

f = c / l

Where f is frequency, c is 300 Mm/s (Speed of light is megameters per second to get megahertz), and l is wavelength.

Eg. 160m band corresponds to what frequency:

300/160 = ~2 (slightly less than 2)

Slightly less than 2, then go lower bc. of 300Mm/s overestimate to get 1.8Mhz - 2.0Mhz.

Note that 300 Mm/s overestimates it, so the actual frequency will be lower than what you calculated.

4th colour band on a 47 ohm resistor gives you resistance tolerance in percent. Obv.

Ferrite Cores are used to minimize RF pickup by audio wires.

In a receiver, an RF amplifier is often used to amplify signals (microvolts!). Single sideband and CW receivers use these.

Marks and Spaces are used to transmit ones and zeroes. From telegraphs, corresponding to dots and dashes.

Quality of receiver is measured by:
Sensitivity, Selectivity, Stability.

$20 per qualification at Gov. of Canada office.

ITU regions:
1 - Europe, Africa, Russia

2 - Americas
3 - Asia

dB Gain Amplifiers:

3dB gain doubles power

6dB gain quadruples power

10dB gain 10x’s power

P_out = P_in ^ (Gain(dB) / 10)

P = P^dB/10

Xc = 1/2pif*C

Use reactance formula to determine how a capacitor reacts to AC current.

Received signal voltage is measured on the S-meter scale (S1-S9). The scale is logarithmic, each accounts for 6dB. On the level of microvolts.

You say a signal is “10 dB over S9”

To add to an existing reported signal, add the dB change.

Eg. “10 dB over S9” if transmitter power is reduced 10x, you get 10dB - 10dB over S9, or just S9.

Yagi Antenna Electromagnetic Wave Polarization:

The electric field oscillates strongest in the plane perpendicular to its length.

Innovation, Science, and Economic Development Canada makes final decisions about antenna systems when there are disputes.

ALC = Automatic Level Control

When it acts, it’s a corrective action

You want signal strong enough to fully modulate the transmitter, but not strong enough that you’re hitting the ALC. So, you want slight movements of the ALC meter on modulation peaks.

Amateur stations are allowed to operate with other authorized stations. No unauthorized stations, but anyone else!

Excessive heat destroy transistors.

SSB (Single Sideband radio) is a type of data encoding.

Audio rectification in the context of radio could mean inadvertently detecting an RF signal, distortion.

PA systems use wires to transmit audio, which means rectification from nearby operating radios could be a problem.

3dB = 2x, 6dB = 4x
Hence, 9dB = 8x

Work in multiples of 3 and multiply!!

Frequency Modulation Receiver:

First, signals from an antenna and amplified

Then, is is mixed and filtered

Before outputting into speaker, it is limited

Then, frequencies discriminated

Then, more amplification to desired voltage amplitude.

Discriminator works best without much noise, so it’s after the limiter.

Advanced qualification means you cannot exceed 1kW power input into a radio transmitter.

You must use minimum power necessary to communicate.

Basic = 250 W DC

Learn the Standard International Phonetic Alphabet

Inductance depends on magnetic field. This depends on core material, coil diameter (electric fields), and number of turns.

Max induced voltage over an inductor happens at max derivative of current.

If impedance of a cable and antenna are mismatched, you need to use an impedance-matching device.

Receiver Overload = early stages of a receiver overwhelmed by a nearby transmitter. Overload = too much incoming signal.

Parasitic Oscillations = unwanted oscillation above or below operating frequency.

This is a subset of spurious emissions, which can be caused by harmonics too.

Antennas should be just out of reach.

<28 MHz allows 6 kHz

10m band allows 20kHz (28.0 - 29.7 MHz)

6m band allows 30 kHz (50-54 MHz)

2m band allows 30kHz

Generally, lower bandwidth allows higher frequency. Higher frequency = lower bandwidth.

Frequency Modulation Transmitter:

Amplifier -> Modulator -> Oscillator -> Frequency Multiplier -> Power Amplifier

The oscillator generates the carrier signal

The modulator alters the frequency

So, the modulator is the input into the oscillator, which takes it then makes the carrier signal.

Eh good enough for these purposes.

Power Amplifier drives the antenna.

Temperature inversion in the upper atmosphere causes ducting of radio waves.

AM radio = Amplitude Modulation to transmit data

Inductive reactance can be increased by increasing frequency.

X_l = wL, w = 2pif, increase in f = increase in X_l

Antenna Changeover switch in a transmitter-receiver combination lets you use an antenna as both a transmitter and a receiver. Antenna Changeover.

Types of propagation:

- Line of Sight

- Sky-wave (bounced off ionosphere)

- Ground-wave (bounced off surface)

Sky-wave gets you to >4000 km

Ionosphere is most ionized at midday and least just before dawn. The sun ionized the ionosphere and it’s a function of direct sunlight.

Many regions of the ionosphere.
D = closest to Earth

E = above D below F

F1, F2 = Only in daytime. Farthest.

,

E = 2000 km

F = >4000 km

Multihop = hop to E and to F

Refraction occurs when waves travel through different densities. This is Tropospheric bending.

In contract, Tropospheric Ducting is caused by temperature inversions.

Tropospheric wave = radiation kept close to the Earth’s surface. Ie. under the troposphere.

Troposphere = lowest level of atmosphere

Meteor scatter, tropospheric scatter, ionospheric scatter are all scatter modes. Distortion.

Meteor scatter is most effective on the 6 meter band.

The ionosphere changes the polarization of a signal. This is important for VHF and UHF, and NOT for lower frequencies (eg. HF 3-30MHz).

RST: Reliability, Signal Strength, Tone Quality. Higher number = better.

1 1 terrible, 5 7 great.

In SSB radio, the frequency of the original modulating signal is conveyed by the position of each side frequency in the sideband in relation to the phantom carrier.

It’s relative to the phantom carrier.

A sideband is a group of ever changing side frequencies. It’s formed by the sum or difference of modulating frequencies and the carrier frequency.

The original frequency from an SSB transmission can be reproduced ebay reinserting a reference signal. A beat frequency oscillator reinserts this reference signal.

Ammeter have very low resistance, don’t resist current if you’re measuring it.

In a regulated power supply, the rectifier is between the transformer and filter.

Q codes:
M = interference
N = static

SWR = measure of impedance matching between transmitter and antenna, efficiency